Data-Enabled Predictive Control: In the Shallows of the DeePC

Jeremy Coulson	John Lygeros	Florian Do¨rfler

arXiv:1811.05890v2 [math.OC] 15 Mar 2019

Abstract— We consider the problem of optimal trajectory tracking for unknown systems. A novel data-enabled predictive control (DeePC) algorithm is presented that computes optimal and safe control policies using real-time feedback driving the unknown system along a desired trajectory while satisfying system constraints. Using a finite number of data samples from the unknown system, our proposed algorithm uses a behavioural systems theory approach to learn a non-parametric system model used to predict future trajectories. The DeePC algorithm is shown to be equivalent to the classical and widely adopted Model Predictive Control (MPC) algorithm in the case of deterministic linear time-invariant systems. In the case of nonlinear stochastic systems, we propose regularizations to the DeePC algorithm. Simulations are provided to illustrate performance and compare the algorithm with other methods.

- Introduction

As systems are becoming more complex and data is becoming more readily available, scientists and practitioners are beginning to bypass classical model-based techniques in favour of data-driven methods [1], [2]. Data-driven methods are suitable for applications where first-principle models are not conceivable (e.g., in human-in-the-loop applications), when models are too complex for control design (e.g., in fluid dynamics), and when thorough modelling and parameter identification is too costly (e.g., in robotics). In fact, it is sometimes easier to learn control policies directly from data, rather than learning a model (the quintessential example supporting this claim is PID control [3]).

A challenging problem in systems control is optimal trajectory tracking, where a control policy is computed based on output feedback that drives a dynamical system along a desired output trajectory while minimizing a stage cost and respecting safety constraints. A special case of the trajectory tracking problem is regulation, in which a control policy drives the system to an equilibrium point. One of the most celebrated and widely used control techniques for trajectory tracking is receding horizon Model Predictive Control (MPC), precisely because it allows one to include safety considerations during control design [4]–[8]. Applica- tions in which trajectory tracking is approached via MPC include autonomous driving [9], autonomous flight [10], mobile robots [11], and smart energy systems [12], among others. The key ingredient for MPC is an accurate parametric state space model of the system (represented by state space matrices), but obtaining such a model is often the most time- consuming and expensive part of control design [13]–[15].

All authors are with the Department of Information Technology and Elec- trical Engineering at ETH Zu¨rich, Switzerland {jcoulson, lygeros, dorfler}@control.ee.ethz.ch. This research was supported by ETH Zu¨rich funds and by the ERC under the project OCAL, grant number 797845.

In the context of unknown black-box systems, there is no approach which solves the optimal trajectory tracking problem subject to constraints and partial (output) observa- tions. However, some more benign variations of the optimal trajectory tracking problem have been approached using data- driven and learning based methods. We do not attempt to provide a comprehensive survey of the recent, albeit already vast, literature. Rather, we single out a few references that are representative for (and at the forefront of) different approaches in the literature.

We begin with reinforcement learning approaches in which control actions are chosen to maximize a reward [16]–[19]. This requires either exploring the system via random control actions, or exploiting the knowledge gained by applying optimal control actions. Reinforcement learning approaches usually require a large number of data samples to perform well, and are often very sensitive to hyper-parameters leading to non-reproducible and highly variable outcomes [20], [21]. Additionally, these approaches do not address all of the challenges present in the optimal trajectory tracking problem; namely, they generally do not take into account safety constraints, and typically assume full state feedback.

Other approaches propose performing sequential system identification (ID) and control. System ID can be used to produce an approximate model as well as provide finite sam- ple guarantees quantifying model uncertainty, thus allowing for robust control design [22]–[24]. In this spirit, an end- to-end ID and control pipeline is given in [25], [26] and arrives at a data-driven control solution with guarantees on the sample efficiency, stability, performance, and robustness. The system identification step in these approaches disregards one of the main advantages of a data-driven approach: inde- pendence from an underlying parametric representation (e.g., state space representation). In fact, non-parametric learning approaches often outperform parametric approaches (see, e.g., Gaussian processes for regression [27]). Additionally, they only consider regulation, rely on having full state information, and do not enforce constraint satisfaction.

Beyond reinforcement learning and sequential ID and control, there are many other adaptive control and safe learning approaches [28], [29]. However, they rely on a-priori stabilizing controllers and safe regions, and thus apply only to a small class of problems.

MPC based on Dynamic Matrix Control has been histori- cally used as a data-driven control technique, in which zero- initial condition step responses are used to predict future trajectories [30], [31]. Although this technique has many limitations [32], it motivates the use of a non-parametric predictive control model. Other non-parametric predictive

models have been proposed in [33]–[35]. These methods do not solve the problem of optimal trajectory tracking with constraints, but serve as building blocks for our approach.

Here we present a Data-enabled Predictive Control (DeePC) algorithm. Unlike classical MPC and the learning- based control techniques outlined above, the DeePC algo- rithm does not rely on a parametric system representation. Instead, similar to [33], we approach the problem from a behavioural systems theory perspective [36]–[38]. Rather than attempting to learn a parametric system model, we aim at learning the system’s “behaviour” (see Section IV for the precise definition). Our novel predictive control strategy

computes optimal controls for unknown systems using real-

In the case when the model for the system is known, i.e., matrices A, B, C and D are known, the problem can be approached using MPC (see Section III). This paper focuses on the above trajectory tracking problem in the case when the model for system (1) is unknown, but input/output data samples are available.

- MPC: A BRIEF OVERVIEW

We outline the well-known receding-horizon model pre- dictive control and estimation algorithm for trajectory track- ing when the model of system (1) is known (see, e.g., [41]). Consider the following optimization problem:

Σ  yk − rt+k

N −1

time output feedback via a receding horizon implementation, allowing one to incorporate input/output constraints to ensure

minimize

u,x,y

2

Q

k=0

+  uk  2

R

safety. We formally show the equivalence of the DeePC algorithm to the classical MPC algorithm in the special case of deterministic linear time-invariant (LTI) systems. Our approach is much simpler to implement than the learning- based techniques above, as well as model-based approaches which require system identification and state observer design.

subject to	xk+1 = Axk + Buk, ∀k ∈ {0, . . . , N − 1},

yk = Cxk + Duk, ∀k ∈ {0, . . . , N − 1},

x0 = xˆ(t),	(2)

uk ∈ U , ∀k ∈ {0, . . . , N − 1},

yk ∈ Y, ∀k ∈ {0, . . . , N − 1},

In contrast to most MPC formulations (where full state

measurement is required), the DeePC algorithm only requires

where N ∈ Z>0

is the time horizon, u = (u0, . . . , u

N −1),

output measurements. Additionally, since our approach does not rely on a parametric system model, we are hopeful that it can also be applied beyond deterministic LTI systems. To apply the algorithm to such systems, we propose some

x = (x0, . . . , xN ), y = (y0, . . . , yN−1) are the decision

variables, and rt+k ∈ Rp is the desired reference at time t+k, where t ∈ Z≥0 is the time at which the optimization problem should be solved. The norm uk  R denotes the quadratic

form uT Ruk (similarly for  · Q), where R ∈ Rm×m is

preliminary though insightful regularizations (e.g., low-rank

approximations [39], and introduction of auxiliary slack

k

the control cost matrix and Q ∈ R

p×p

is the output cost

variables [40]), and reason as to why these regularizations improve performance. All of the results are validated with a nonlinear and stochastic aerial robotics case study, in which the DeePC algorithm is shown to outperform sequential ID with MPC.

The remainder of the paper is organized as follows: in Section II, we formally define the problem. In Section III we review the basic notion of MPC. In Section IV we introduce behavioural system theory. Section V contains the DeePC algorithm. In Section VI we simulate the DeePC algorithm and compare it with sequential ID and MPC using a quadcopter simulation. We conclude the paper in Section VII.

- Problem Statement

(

Consider the discrete-time system given by

matrix. The estimated state at time t is denoted by xˆ(t) and

the predicted state and output at time t + k are denoted by xk and yk, respectively. If the entire state is measured then xˆ(t) = x(t). When the state measurement is not available, but system (1) is observable, an observer is typically used to estimate the state based on knowledge of the system (1) and the measured output y [7]. One may also combine the control and estimation into a single min-max optimization problem [42].

The classical MPC algorithm involves solving optimiza- tion problem (2) in a receding horizon manner.

Algorithm 1 MPC

Input: (A, B, C, D), reference trajectory r, past input/output data (u, y), constraint sets U and Y, and performance matri- ces Q and R

x(t + 1) = Ax(t) + Bu(t)

y(t) = Cx(t) + Du(t),

(1)

- Generate state estimate xˆ(t) using past input/output data.

- Solve (2) for u⋆ = (u⋆, . . . , u⋆	).

0	N −1

n×n

n×m

p×n

p×m

- Apply inputs (u(t), . . . , u(t + s)) = (u⋆, . . . , u⋆) for

where A ∈ R	, B ∈ R	, C ∈ R	, D ∈ R	, and	0	s

x(t) ∈ Rn, u(t) ∈ Rm, y(t) ∈ Rp are respectively the state, control input, and output of the system at time t ∈ Z≥0. Given a desired reference trajectory r = (r0, r1, . . . ) ∈ (Rp)Z≥0 , input constraint set U ⊆ Rm, output constraint set Y ⊆ Rp, we wish to apply control inputs such that the system output tracks the reference trajectory r while satisfying

constraints and optimizing a cost function. Tracking of the trivial trajectory r = (0, 0, . . . ) is simply regulation.

some s ≤ N − 1.

- Set t to t + s and update past input/output data.

- Return to 1.

Note that choosing s > 0 reduces the number of compu- tations, and in some cases may improve performance [43].

Under standard assumptions, it can be shown that the MPC algorithm is recursively feasible and stabilizing [5].

One crucial ingredient for MPC is an accurate model of

and T ′ ∈ Z>0 such that wt = w1

for 1 ≤ t ≤ T and

t

the system; this is needed both to formulate problem (2)

2

wt = w

t−T −T

′ for t > T + T ′.

and, in cases where the state is not measured exactly, to

generate the initial state estimates xˆ(t). The need for a model is traditionally addressed through system identification [44], where observations of the system are collected offline before the online control operation begins and are used to estimate a model of the form (1) that matches the observed data in an appropriate sense. For complex systems, this can

be a cumbersome and expensive process [13]–[15]. For

In other words, a behavioural system is controllable if any two trajectories can be patched together in finite time.

Definition 4.4: Let L, T ∈ Z>0 such that T ≥ L. The signal u = col(u1, . . . , uT ) ∈ RTm is persistently exciting of order L if the Hankel matrix

L	

.

. . .	.



	u1	· · ·	uT −L+1

H (u) :=

this reason, we propose a Data-enabled Predictive Control

(DeePC) algorithm which learns the behaviour of the system and does not require an explicit model, system identification,

is of full row rank.

uL	· · ·	uT

or state estimation (see Algorithm 2).

- Preliminaries

  - Non-parametric system representation

Behavioural system theory is a natural way of viewing a dynamical system when one is not concerned with a particular system representation, but rather the subspace of the signal space in which trajectories of the system live. This is in contrast with classical system theory, where a particular parametric system representation (such as the state-space model (1)) is used to describe the input/output behaviour, and properties of the system are derived by studying the chosen system representation. Following [38], we define a dynamical system and its properties in terms of its behaviour.

Definition 4.1: A dynamical system is a 3-tuple (Z≥0, W, B) where Z≥0 is the discrete-time axis, W is a signal space, and B ⊆ WZ≥0 is the behaviour.

Definition 4.2: Let (Z≥0, W, B) be a dynamical system.

    - (Z≥0, W, B) is linear if W is a vector space and B is a linear subspace of WZ≥0 .

    - (Z≥0, W, B) is time invariant if B ⊆ σB where

The term persistently exciting describes an input signal sufficiently rich and long as to excite the system yielding an output sequence that is representative for the system’s behaviour.

  - Parametric system representation

There are several equivalent ways of representing a behavioural system B ∈ Lm+p, including the clas- sical input/output/state representation (1) denoted by B(A, B, C, D) = {col(u, y) ∈ (Rm+p)Z≥0 | ∃ x ∈ (Rn)Z≥0  s.t. σx = Ax + Bu,  y = Cx + Du}. The in-

put/output/state representation of smallest order (i.e., smallest

state dimension) is called a minimal representation, and we denote its order by n(B). Another important property of a system B ∈ Lm+p is the lag defined by the small- est integer ℓ ∈ Z>0 such that the observability matrix

Oℓ(A, C) := col C, CA, . . . , CAℓ−1  has rank n(B). We

denote the lag by ℓ(B) (see [38, Section 7.2] for equivalent input/output/state representation free definitions of lag). The lower triangular Toeplitz matrix consisting of A, B, C, D is denoted by

σ : WZ≥0 → WZ≥0 is the forward time shift defined

by (σw)(t) = w(t + 1) and σB = {σw | w ∈ B}.

	D	0	· · ·	0 

≥0

of pointwise convergence.

N

(iii) (Z

, W, B) is complete if B is closed in the topology

T  (A, B, C, D) := 

CB	D	· · ·	0



.	.	. 



.

. .	. .	.

.

Note that if a dynamical system satisfies (i)-(ii) then (iii) is equivalent to finite dimensionality of W (see [38, Section 7.1]). We denote the class of systems (Z≥0, Rm+p, B) satisfying (i)-(iii) by Lm+p, where m, p ∈ Z≥0. With slight abuse of notation and terminology, we denote a dynamical

CAN−2B	· · ·	CB	D

We can now present a uniqueness result.

Lemma 4.1: ([33, Lemma 1]): Let B ∈ Lm+p and B(A, B, C, D) a minimal input/output/state representation. Let Tini, N ∈ Z>0 with Tini ≥ ℓ(B) and col(uini, u, yini, y) ∈

system in Lm+p only by its behaviour B.

Next, we define the set BT = {w ∈ (Rm+p)T | ∃ v ∈

B s.t. wt = vt, 1 ≤ t ≤ T } of trajectories truncated

BTini

+N . Then there exists a unique xini ∈ Rn(B) such that

y = ON (A, C)xini + TN (A, B, C, D)u.	(3)

to a window of length T . Without loss of generality, we

assume that B can be written as the product space of two sub-behaviours Bu and By, where Bu = (Rm)Z≥0 and By ⊆ (Rp)Z≥0 are the spaces of input and output signals, respectively (see [36, Theorem 2]), that is, any trajectory w ∈ B can be written as w = col(u, y), where col(u, y) := (uT , yT )T . We now present two concepts: controllability, and persistency of excitation.

Definition 4.3: A system B ∈ Lm+p is controllable if for every T ∈ Z>0, w1 ∈ BT , w2 ∈ B there exists w ∈ B

In other words, given a sufficiently long window of initial

system data col(uini, yini), the state to which the system is driven by the sequence of inputs uini is unique. Furthermore, if the matrices A, B, C, D are known, the state xini can be computed. We now present a result known in behavioural systems theory as the Fundamental Lemma [33].

Lemma 4.2: ([37, Theorem 1]): Consider a controllable system B ∈ Lm+p. Let T, t ∈ Z>0, and w = col(u, y) ∈ BT . Assume u to be persistently exciting of order t+n(B). Then colspan(Ht(w)) = Bt.

Note that the number of data points T must be at least (m + 1)(t + n(B)) − 1 in order to satisfy the persistency of excitation condition. Lemma 4.2 replaces the need for

a model or system identification process and allows for any trajectory of a controllable LTI system to be con- structed using a finite number of data samples generated by a sufficiently rich (in particular, persistently exciting) input signal. In a sense, the Hankel matrix is itself a non- parametric predictive model based on raw data. It allows one to implicitly estimate the state of an LTI system, predict its future behaviour, and design optimal feedforward control inputs [33]. Lemma 4.1 and Lemma 4.2 can be used together in a predictive control algorithm similar to Algorithm 1 by replacing the state observer (respectively, the system model) with the data col(uini, yini) (respectively, Ht(w)).

- DeePC: A data-enabled predictive control algorithm

- Data collection

We begin by assuming that the data is generated by an

fixes the underlying initial state xini from which the trajectory col(u, y) evolves. Note, however, that (5) does not require the input/output/state representation of the system to be known. The state xini is only “fixed” implicitly by col(uini, yini). As first shown in [33], this allows one to predict future trajectories based on a given initial trajectory col(uini, yini) ∈

BTini , and the precollected data in Up, Uf , Yp, and Yf. Indeed,

given an initial trajectory col(uini, yini) ∈ BTini of length Tini ≥ ℓ(B) and a sequence of future inputs u ∈ RNm, the first three block equations of (5) can be solved for g. The sequence of future outputs are then given by y = Yf g.

Furthermore, by Lemma 4.1 the vector y computed contains the unique sequence of outputs corresponding to the inputs u. Vice versa, given a desired reference output y an associated feedforward control input can be calculated [33].

  - DeePC algorithm

Given a time horizon N ∈ Z>0, a reference trajec- tory r = (r0, r1, . . . ) ∈ (Rp)Z≥0 , past input/output data

col(u  , y  ) ∈ B  , input constraint set U ⊆ Rm, output

unknown controllable LTI system B ∈ Lm+p with a min-

ini

ini

Tini

p

p×p

imal  input/output/state  representation  B(A, B, C, D).  Let

constraint set Y ⊆ R , output cost matrix Q ∈ R

, and

T, T

ini

, N ∈ Z>0

such that T ≥ (m+1)(T

ini

+N +n(B))−1.

control cost matrix R ∈ Rm×m, we formulate the following

optimization problem:

Let ud = col(ud , . . . , ud ) ∈ RTm be a sequence of T inputs

1	T

applied to B, and yd = (yd, . . . , yd ) ∈ RTp the correspond-

N −1

ing outputs. Furthermore, assume u

is persistently exciting

minimize

1	T d	Σ

2		 2

	

of order Tini+N +n(B). The superscript d is used to indicate that these are sequences of data samples collected during an offline procedure from the unknown system. Note that the data col(ud, yd) ∈ BT can be equivalently thought of as

g,u,y

subject to

k=0

yk − rt+k

Q +  uk

R

	

Up	uini

			

Yp	g =	yini	,	(6)

Uf 	 u 

coming from the minimimal input/output/state representation

B(A, B, C, D). Next, we partition the input/output data into two parts which we call past data and future data. More formally, define

Uf

ini

Yf

ini

+	T +N

Up  := HT	N (ud),	 Yp  := H	(yd),	(4)

Yf

y

uk ∈ U , ∀k ∈ {0, . . . , N − 1},

yk ∈ Y, ∀k ∈ {0, . . . , N − 1}.

Note here, that u and y are not independent decision variables

of the optimization problem. Rather they are described

where Up consists of the first Tini block rows of HT +N (ud)

completely by the fixed data matrices Uf and Yf and the

and U

ini

consists of the last N block rows of H

d	decision variable g. A comparison of the two optimal control

f	Tini+N (u )

(similarly for Yp and Yf). In the sequel, past data denoted by the subscript p will be used to estimate the initial condition of the underlying state, whereas the future data denoted by the subscript f will be used to predict the future trajectories.

- State estimation and trajectory prediction

By Lemma 4.2, we can construct any Tini + N length trajectory of BTini+N using the data collected. Indeed, a

problems (2) and (6) yields only a single (though key) difference; the model and the state estimate in (2) are replaced completely with input/output data samples in (6). We now present the DeePC algorithm.

Algorithm 2 DeePC

Input: col(ud, yd) ∈ BT , reference trajectory r ∈ RNp, past input/output data col(uini, yini) ∈ BTini , constraint sets U and

trajectory col(uini, u, yini, y) belongs to BTini+N if and only if there exists g ∈ RT −Tini−N+1 such that

Y, and performance matrices Q and R

- Solve (6) for g⋆.

Up

uini

- Compute the optimal input sequence u⋆ = Ufg⋆.

- Apply input (u(t), . . . , u(t + s)) = (u⋆, . . . , u⋆) for

			

Yp	yini

	

g =	.	(5)

0	s

some s ≤ N − 1.

	

Uf	u

y

Yf

- Set t to t + s and update u

ini

and y

ini

to the T

ini

most

If Tini ≥ ℓ(B), then Lemma 4.1 implies that there exists a unique xini ∈ Rn(B) such that the output y is uniquely determined by (3). Intuitively, the trajectory col(uini, yini)

recent input/output measurements.

- Return to 1.

D. Equivalence of DeePC and MPC

(by Theorem 5.1), then u⋆

⋆

= u

DeePC

. Applying control

MPC

inputs (u(t), . . . , u(t + s)) = (u⋆, . . . , u⋆) for some s ≤

It can be shown that Algorithm 1 and Algorithm 2 yield	0	s

equivalent closed-loop trajectories under certain assumptions.

N − 1 to system (1) yields corresponding output sequence

(y(t), . . . , y(t + s) = (y⋆, . . . , y⋆). Updating col(uini, yini) to

We first show the equivalence of the feasible sets of (2)	0	s

and (6).

Theorem 5.1: (Feasible Set Equivalence): Consider a controllable LTI system B ∈ Lm+p with minimal in- put/ouput/state representation B(A, B, C, D) given as in (1). Consider the MPC and DeePC optimization problems (2)

and (6). Let Tini ≥ ℓ(B) and col(uini, yini) ∈ BTini  be

the most recent input/output measurements and setting the

state estimate in Algorithm 1 to xini as in Theorem 5.1 yields equal feasible sets. Repeating the above argument, both algorithms compute an identical optimal control sequence. This argument can be repeated for all iterations of the algorithms proving the result.

the most recent input/output measurements from system (1). Assume that the data col(ud, yd) ∈ BT in col(Up, Yp, Uf, Yf ) is such that ud is persistently exciting of order Tini + N + n(B), where T ≥ (m + 1)(Tini + N + n(B)) − 1. Then there exists a state estimate xˆ(t) in (2) such that the feasible sets of (2) and (6) are equal.

Proof: We first look at the feasible set of (6). Since ud is persistently exciting of order Tini + N + n(B), then by Lemma 4.2 image (col(Up, Yp, Uf, Yf )) = BT +N , where

One notable feature of the DeePC algorithm presented in

Algorithm 2 is its simplicity when compared to reinforce- ment learning approaches [20], and other related model- based schemes. The DeePC algorithm achieves system ID, state estimation, and trajectory prediction with one linear

equation resulting in a quadratic program with T −Tini−N +1 number of decision variables where T ∈ Z>0 is the amount of data collected. In order to satisfy the persistency of

excitation assumption in Theorem 5.1, one must collect a

ini

Up, Yp, Uf, Yf are defined as in (4). Hence, the feasible set

minimum of (m + 1)(T

ini

+ N + n(B)) − 1 data samples

of (6) is equal to {(u, y) ∈ UN × YN | col(uini, u, yini, y) ∈

implying that the number of decision variables in (6) is at

BTini

+N }, where UN is the cartesian product of U with itself

least m(Tini + N ) + (m + 1)n(B).

N -times (similarly for YN ). Since the system B yields an equivalent representation given by B(A, B, C, D), then by Lemma 4.1 the feasible set of (6) can be written as the set of pairs (u, y) ∈ UN × YN satisfying

y = ON (A, C)xini + TN (A, B, C, D)u,

where xini is uniquely determined from col(uini, yini). We now look at the feasible set of (2). By rewriting the con- straints in (2) we obtain

y = ON (A, C)xˆ(t)+TN (A, B, C, D)u,	u ∈ UN , y ∈ YN ,

where xˆ(t) is the estimation of the state x(t) at time t. Setting the state estimate xˆ(t) = xini yields equal feasible sets since the state-space coordinates of the B(A, B, C, D) and the system in (2) are identical.

Note that the state estimate xˆ(t) = xini is a natural choice when full-state measurements are available or when input/output measurements are deterministic.

Corollary 5.1: (Equivalent Closed Loop Behaviour): Consider the MPC Algorithm 1 and the DeePC Algorithm 2 with Q ≥ 0, R ≻ 0, and U, Y convex and non-empty. Under the assumptions of Theorem 5.1, Algorithm 1 and Algorithm 2 result in equivalent closed-loop behaviour, i.e., the optimal control sequence u⋆ and corresponding system

output y⋆ at every iteration is identical.

, y

⋆

Proof: Since R ≻ 0, the cost function in (2) is strictly convex in the decision variable u. Thus, since the constraints

Note that the persistency of excitation assumption assumes

knowledge of n(B) and ℓ(B), which are properties that are a priori unknown. We know that ℓ(B) ≤ n(B) by the Cayley-Hamilton theorem. Hence an upper bound for n(B) is sufficient. In practice, one would simply collect a sufficiently large amount of data to exceed the necessary amount for persistency of excitation. If, however, n(B) is underestimated and an insufficient amount of data is collected, the matrix col(Up, Yp, Uf, Yf) will represent a reduced order linear system with an approximate input/output

behaviour. The precise implications of this model reduction need to be investigated in future work.

- Beyond deterministic LTI systems

In this section we provide preliminary results which shows the promising extension of the DeePC algorithm beyond deterministic LTI systems. We offer insightful algorithmic extensions by means of salient regularizations, show their utility through a numerical study, and provide plausible reasoning for the regularizations.

- Regularized DeePC Algorithm

(

Consider now the nonlinear discrete-time system given by

x(t + 1) = f (x(t), u(t))	(7)

y(t) = h(x(t), u(t), η(t)),

where η(t) ∈ Rp is white measurement noise, and f : Rn ×

MPC

are convex and non-empty, a solution (u⋆

⋆

, x

MPC

MPC)

Rm → Rn and h : Rn × Rm × Rp → Rp are not necessarily

MPC

to (2) exists, and u⋆

is unique (see, e.g., [45]). Similarly,

linear. One may also consider a system affected by process

the cost function in (6) is strictly convex in the decision variable u and the constraints are convex and non-empty.

noise. However, we focus on systems only affected by measurement noise in order to isolate its effect on the DeePC

DeePC

Hence, a solution (g⋆

⋆

, u

DeePC

⋆

, y

DeePC

) to (6) exists, and

algorithm. To apply the DeePC algorithm to system (7), we

u

⋆

DeePC

is unique. Since the cost function in (2) and (6)

propose three regularizations to the optimal control prob-

coincide and the feasible sets of (2) and (6) are equal	lem (6). In particular, we introduce the following regularized

optimization problem:

Σ  yk − rt+k

N −1

compare the performance to system identification (ID) fol- lowed by MPC using the identified model. The states of

minimize

g,u,y,σy

2

Q

k=0

+  uk  2

the quadcopter model are given by the 3 spatial coor- dinates (x, y, z) and their velocities, and the 3 angular

R

+ λg  g  1 + λy  σy  1

coordinates (α, β, γ) and their velocities, i.e., the state is

˙

U^p

uini

 0 

(x, y, z, x˙, y˙, z˙, α, β, γ, α˙ , β, γ˙). The inputs are given by the thrusts from the 4 rotors, (u1, u2, u3, u4). We assume full

subject to

Y^p  g = yini  + σy ,	(8)

state measurement to facilitate the comparison to standard

Y^f

uk ∈ U , ∀k ∈ {0, . . . , N − 1},

U^f 

 u 

y

 0 

0

MPC. Data was collected from the nonlinear model subject to additive white measurement noise. We collected 214 in-

put/output measurements with a sample time of 0.1 seconds.

yk ∈ Y, ∀k ∈ {0, . . . , N − 1},

^	^  ^  ^

where σy ∈ RTinip is an auxiliary slack variable, λy, λg ∈ R>0 are regularization parameters, and col(Up, Yp, Uf, Yf ) is a low-rank matrix approximation of col(Up, Yp, Uf, Yf ). Let

us explain these three regularizations. We offer convincing numerical evidence in Section VI-B.

Slack variable: When the output measurements are cor- rupted by noise, the constraint equation in (6) may become inconsistent. Hence, in (8) we include the slack variable σy in the constraint to ensure feasibility of the constraint at all times. We penalize the slack variable with a weighted one-norm penalty function. Choosing λy sufficiently large

gives the desired property that σy /= 0 only if the constraint is infeasible (see, e.g., [40]), that is, only if the data is

inconsistent.

One-norm regularization on g: The cost includes a one- norm penalty on g. We conjecture that this regularization is related to distributionally robust optimization problems, in which similar regularization terms arise [46], [47].

^	^  ^  ^

Low-rank approximation: By performing a low-rank matrix approximation (e.g., via singular value decomposition (SVD) and truncation [39]), we take into account only the most dominant sub-behaviour (corresponding to the largest singular values), resulting in a data matrix describing the behaviour of the closest deterministic LTI system (where “closest” is measured with the Frobenius norm in the SVD case). In the case of noisy measurements, the SVD filters the noise. In the case of nonlinear dynamics (which can be lifted to infinite-dimensional linear dynamics with a nonlinear output map [48], [49]), the SVD results in a matrix describing an approximate LTI model, i.e., the most relevant basis functions of the infinite-dimensional lift whose dimension can be chosen by adjusting the SVD cutoff. Note that after performing the low-rank approximation, the DeePC algorithm does not require that the matrix col(Up, Yp, Uf, Yf ) have a Hankel structure. This is in contrast to subspace ID techniques, in which low-rank approximations must be care- fully modified in order to preserve the Hankel structure of the data matrix resulting in higher algorithmic and computational complexity [50].

- Aerial Robotics Case Study

We illustrate the performance of the regularized DeePC algorithm, i.e., Algorithm 2 with (8), by simulating it on a high-fidelity nonlinear quadcopter model [51], and

Drawing the input sequence from a uniformly distributed

^	^  ^  ^

random variable ensured that the data was persistently ex- citing. For the model-based MPC, the data was used to identify the system parameters through the least square prediction error method with an assumed state dimension of 12. The following parameters were chosen for the opti- mization problems (2) and (8): N = 30, Tini = 1, R = I, Q = diag(200, 200, 300, 1, . . . , 1), λg  = 30, λy = 105,

col(Up, Yp, Uf, Yf ) = col(Up, Yp, Uf, Yf ). The thrust from each rotor was constrained between 0 and 1, and the (x, y, z) coordinates were constrained between −3 and 3.

We simulated the system ID followed by the MPC algo-

rithm and the regularized DeePC algorithm on the nonlinear and stochastic quadcopter model in which the quadcopter was commanded to follow a series of figure-eight trajectories for a duration of 60 seconds. We observe that DeePC performs better than sequential ID and MPC in terms of reference tracking and constraint satisfaction; see Figure 1 for an illustration.

Another simulation was performed in which the quad- copter was commanded to perform a simple step trajectory in the (x, y, z) coordinates with the same constraints listed above. The duration of constraint violations and the cost were measured. This was repeated 30 times with different data sets for constructing col(Up, Yp, Uf, Yf), and different random seeds for the measurement noise. The results are displayed in Figure 2 and show that DeePC consistently outperforms identification-based MPC in terms of cost and constraint satisfaction. While these observations should be made cautiously, an intuitive explanation for the superior performance of DeePC is that (8) simultaneously optimizes for the best system model, state estimation, and control policy, whereas conventional MPC requires fixing a system model and performs these tasks independently.

To study the effect of the regularizations on the perfor- mance of the DeePC algorithm, we performed a sensitivity analysis on the regularization parameters λy and λg. We did not perform any low-rank approximation to the data. The quadcopter was commanded to follow the same step trajectory as in the previous simulation. The duration of constraint violations and cost were measured. This was repeated 8 times with different data sets, and the duration of constraint violations and cost were averaged over these 8 data sets. The results in Figure 3 show that the regularizations improve performance.

Our preliminary simulations suggest that one-norm reg-

ularization of λg is more effective and robust than a low- rank approximation of the Hankel matrix col(Up, Yp, Uf, Yf ). In fact, the latter appears to be sensitive and needs to be performed on a case by case basis to avoid unstable behaviour. This will be investigated in future work.

1010

Cost

108

106

Cost

Constraint Violations

Duration violations (s)

20

15

10

5

0

DeePC

xDeePC

y

DeePC

z

DeePC

xref

y

ref

z

ref

Constraints

3

100	102	104	106

100	102	104	106

2

  - Fixed λg

= 300

  - Fixed λg

= 300

2

1.5

1

0.5

0

-0.5

-1

-1.5

1

m

0

-1

-2

-3

0	10	20	30	40	50	60

s

(b)

MPC

xMPC

y

MPC

z

MPC

xref

y

ref

z

ref

Constraints

5

4

7

107

Cost

6

5

Cost

4

3

2

1

0

0	200	400	600	800

y

Fixed λ = 105

Constraint Violations

Duration violations (s)

20

15

10

5

0

0	200	400	600	800

y

Fixed λ = 105

1

0.5

0	0 -0.2

0.4

-0.5	0.2

3

2

Fig. 3: Performance of quadcopter controlled by the DeePC

algorithm with different regularization parameters.

-1

(a)

0.6

1

m

0

-1

-2

-3

0	10	20	30	40	50	60

s

(c)

- Conclusion

We presented a data-enabled algorithm that can be applied to unknown LTI systems and formally showed its equivalence to the classical MPC algorithm. The DeePC algorithm uses a

Fig. 1: Figure (a): three dimensional plot of the trajectory of

the quadcopter at different instances of time controlled with DeePC. Figure (b) and (c): trajectories of the spatial coor- dinates when controlled by DeePC and MPC, respectively. The horizontal red dashed lines represent constraints.

Constraint Violations

DeePC

System ID + MPC

20

Number of simulations

15

10

5

0

0	2	4	6	8	10	12	14	16	18	20

Duration constraints violated

Cost

DeePC

System ID + MPC

30

Number of simulations

25

20

15

10

5

0	0.5	1	1.5	2

Cost	 107

Fig. 2: Constraint violations and cost comparison of DeePC and system ID with MPC.

finite data set to learn the behaviour of the unknown system and computes optimal controls using real-time feedback to drive the system along a desired trajectory while respecting system constraints. Furthermore, we simulated a regularized version of the algorithm on stochastic nonlinear quadcopter dynamics illustrating its capabilities beyond deterministic LTI systems. The performance was superior when compared to system ID followed by MPC. Ongoing and future work focuses on the robustness of the DeePC algorithm and its regularization when applied to stochastic and nonlinear dynamics.

Acknowledgements

We thank Bassam Bamieh, Ben Recht, Ashish Cherukuri, and Manfred Morari for useful discussions, and Melanie Zeilinger and Colin Jones for providing the simulation model.

References

- F. Lamnabhi-Lagarrigue, A. Annaswamy, S. Engell, A. Isaksson,

P. Khargonekar, R. M. Murray, H. Nijmeijer, T. Samad, D. Tilbury, and P. Van den Hof, “Systems & control for the future of humanity, re- search agenda: Current and future roles, impact and grand challenges,” Annual Reviews in Control, vol. 43, pp. 1–64, 2017.

- Z. S. Hou and Z. Wang, “From model-based control to data-driven control: Survey, classification and perspective,” Information Sciences, vol. 235, pp. 3–35, 2013.

- K. J. A˚ stro¨m and T. Ha¨gglund, PID Controllers: Theory, Design, and Tuning. Instrument society of America, 1995, vol. 2.

- J. B. Rawlings and D. Q. Mayne, Model Predictive Control: Theory and Design. Nob Hill Publishing, 2009.

- E. F. Camacho and C. B. Alba, Model Predictive Control. Springer Science & Business Media, 2013.

- A. Bemporad and M. Morari, “Robust model predictive control: A survey,” in Robustness in identification and control. Springer, 1999,

pp. 207–226.

- D. Q. Mayne, “Model predictive control: Recent developments and future promise,” Automatica, vol. 50, no. 12, pp. 2967–2986, 2014.

- F. Borrelli, A. Bemporad, and M. Morari, Predictive Control for Linear and Hybrid Systems. Cambridge University Press, 2017.

- M. Brown, J. Funke, S. Erlien, and J. C. Gerdes, “Safe driving envelopes for path tracking in autonomous vehicles,” Control Engi- neering Practice, vol. 61, pp. 307–316, 2017.

- I. Prodan, S. Olaru, R. Bencatel, J. B. de Sousa, C. Stoica, and S.-I. Niculescu, “Receding horizon flight control for trajectory tracking of autonomous aerial vehicles,” Control Engineering Practice, vol. 21, no. 10, pp. 1334–1349, 2013.

- F. Bullo, E. Frazzoli, M. Pavone, K. Savla, and S. L. Smith, “Dynamic vehicle routing for robotic systems,” Proceedings of the IEEE, vol. 99, no. 9, pp. 1482–1504, 2011.

- G. Darivianakis, A. Georghiou, R. S. Smith, and J. Lygeros, “The power of diversity: Data-driven robust predictive control for energy- efficient buildings and districts,” IEEE Transactions on Control Sys- tems Technology, 2017.

- H. Hjalmarsson, “From experiment design to closed-loop control,”

Automatica, vol. 41, no. 3, pp. 393–438, 2005.

- B. A. Ogunnaike, “A contemporary industrial perspective on process control theory and practice,” Annual Reviews in Control, vol. 20, pp. 1–8, 1996.

- J. Richalet, “Industrial applications of model based predictive control,”

Automatica, vol. 29, no. 5, pp. 1251–1274, 1993.

- F. L. Lewis, D. Vrabie, and K. G. Vamvoudakis, “Reinforcement learning and feedback control: Using natural decision methods to design optimal adaptive controllers,” IEEE Control Systems, vol. 32, no. 6, pp. 76–105, 2012.

- Y. Ouyang, M. Gagrani, and R. Jain, “Learning-based control of unknown linear systems with Thompson sampling,” arXiv preprint arXiv:1709.04047, 2017.

- B. Kiumarsi, F. L. Lewis, H. Modares, A. Karimpour, and M.-

B. Naghibi-Sistani, “Reinforcement Q-learning for optimal tracking control of linear discrete-time systems with unknown dynamics,” Automatica, vol. 50, no. 4, pp. 1167–1175, 2014.

- A. M. Devraj and S. Meyn, “Zap Q-learning,” in Advances in Neural Information Processing Systems, 2017, pp. 2235–2244.

- B. Recht, “A tour of reinforcement learning: The view from continuous control,” arXiv preprint arXiv:1806.09460, 2018.

- R. Islam, P. Henderson, M. Gomrokchi, and D. Precup, “Reproducibil- ity of benchmarked deep reinforcement learning tasks for continuous control,” arXiv preprint arXiv:1708.04133, 2017.

- M. C. Campi and E. Weyer, “Finite sample properties of system identification methods,” IEEE Transactions on Automatic Control, vol. 47, no. 8, pp. 1329–1334, 2002.

- M. Vidyasagar and R. L. Karandikar, “A learning theory approach to system identification and stochastic adaptive control,” in Probabilistic and randomized methods for design under uncertainty. Springer, 2006, pp. 265–302.

- S. Tu, R. Boczar, A. Packard, and B. Recht, “Non-asymptotic analysis of robust control from coarse-grained identification,” arXiv preprint arXiv:1707.04791, 2017.

- R. Boczar, N. Matni, and B. Recht, “Finite-data performance guar- antees for the output-feedback control of an unknown system,” arXiv preprint arXiv:1803.09186, 2018.

- S. Dean, H. Mania, N. Matni, B. Recht, and S. Tu, “On the sample complexity of the linear quadratic regulator,” arXiv preprint arXiv:1710.01688, 2017.

- C. E. Rasmussen, “Gaussian processes in machine learning,” in

Advanced lectures on machine learning. Springer, 2004, pp. 63–71.

- F. Berkenkamp, M. Turchetta, A. Schoellig, and A. Krause, “Safe model-based reinforcement learning with stability guarantees,” in Advances in Neural Information Processing Systems, 2017, pp. 908– 918.

- J. F. Fisac, A. K. Akametalu, M. N. Zeilinger, S. Kaynama, J. Gillula, and C. J. Tomlin, “A general safety framework for learning-based control in uncertain robotic systems,” IEEE Transactions on Automatic Control, 2018.

- R. W. Freedman and A. Bhatia, “Adaptive dynamic matrix control: Online evaluation of the DMC model coefficients,” in American Control Conference. IEEE, 1985, pp. 220–225.

- C. E. Garcia, D. M. Prett, and M. Morari, “Model predictive control: Theory and practice – A survey,” Automatica, vol. 25, no. 3, pp. 335– 348, 1989.

- P. Lundstro¨m, J. H. Lee, M. Morari, and S. Skogestad, “Limitations of dynamic matrix control,” Computers & Chemical Engineering, vol. 19, no. 4, pp. 409–421, 1995.

- I. Markovsky and P. Rapisarda, “Data-driven simulation and control,”

International Journal of Control, vol. 81, no. 12, pp. 1946–1959, 2008.

- W. Favoreel, B. De Moor, P. Van Overschee, and M. Gevers, “Model- free subspace-based LQG-design,” in American Control Conference, vol. 5. IEEE, 1999, pp. 3372–3376.

- A. Agarwal, M. J. Amjad, D. Shah, and D. Shen, “Time series analysis via matrix estimation,” arXiv preprint arXiv:1802.09064, 2018.

- J. C. Willems, “From time series to linear system – Part I. Finite dimensional linear time invariant systems.” Automatica, vol. 22, no. 5,

pp. 561–580, 1986.

- J. C. Willems, P. Rapisarda, I. Markovsky, and B. L. De Moor, “A note on persistency of excitation,” Systems & Control Letters, vol. 54, no. 4, pp. 325–329, 2005.

- I. Markovsky, J. C. Willems, S. Van Huffel, and B. De Moor, Exact and Approximate Modeling of Linear Systems: A Behavioral Approach. SIAM, 2006.

- S. Chatterjee, “Matrix estimation by universal singular value thresh- olding,” The Annals of Statistics, vol. 43, no. 1, pp. 177–214, 2015.

- R. Fletcher, Practical Methods of Optimization. John Wiley & Sons, 2013.

- A. Ferramosca, D. Limo´n, I. Alvarado, T. Alamo, and E. F. Camacho, “MPC for tracking with optimal closed-loop performance,” Automat- ica, vol. 45, no. 8, pp. 1975 – 1978, 2009.

- D. A. Copp and J. P. Hespanha, “Simultaneous nonlinear model predictive control and state estimation,” Automatica, vol. 77, pp. 143– 154, 2017.

- L. Gru¨ne, J. Pannek, M. Seehafer, and K. Worthmann, “Analysis of unconstrained nonlinear MPC schemes with time varying control horizon,” SIAM Journal on Control and Optimization, vol. 48, no. 8,

pp. 4938–4962, 2010.

- L. Ljung, System Identification: Theory for the User. Prentice-hall, 1987.

- S. Boyd and L. Vandenberghe, Convex Optimization. Cambridge university press, 2004.

- P. M. Esfahani and D. Kuhn, “Data-driven distributionally robust optimization using the wasserstein metric: Performance guarantees and tractable reformulations,” Mathematical Programming, vol. 171, no. 1-2, pp. 115–166, 2018.

- S. Shafieezadeh-Abadeh, D. Kuhn, and P. M. Esfahani, “Regularization via mass transportation,” arXiv preprint arXiv:1710.10016, 2017.

- S. Boyd and L. Chua, “Fading memory and the problem of approx- imating nonlinear operators with Volterra series,” IEEE Transactions on circuits and systems, vol. 32, no. 11, pp. 1150–1161, 1985.

- K. Kowalski and W.-H. Steeb, Nonlinear Dynamical Systems and Carleman Linearization. World Scientific, 1991.

- I. Markovsky, Low-Rank Approximation: Algorithms, Implementation, Applications, 2nd ed. Springer, 2018.

- R. Mahony, V. Kumar, and P. Corke, “Multirotor aerial vehicles,” IEEE Robotics and Automation magazine, vol. 20, no. 32, 2012.