Data-Driven Model Predictive Control With Stability and Robustness Guarantees

Julian Berberich  , Johannes Köhler  , Matthias A. Müller  , and Frank Allgöwer

Abstract—We propose a robust data-driven model pre- dictive control (MPC) scheme to control linear time- invariant systems. The scheme uses an implicit model de- scription based on behavioral systems theory and past measured trajectories. In particular, it does not require any prior identification step, but only an initially measured input–output trajectory as well as an upper bound on the order of the unknown system. First, we prove exponential stability of a nominal data-driven MPC scheme with ter- minal equality constraints in the case of no measurement noise. For bounded additive output measurement noise, we propose a robust modification of the scheme, including a slack variable with regularization in the cost. We prove that the application of this robust MPC scheme in a multistep fashion leads to practical exponential stability of the closed loop w.r.t. the noise level. The presented results provide the first (theoretical) analysis of closed-loop properties, result- ing from a simple, purely data-driven MPC scheme.

Index Terms—Data-driven control, predictive control for linear systems, robust control, uncertain systems.

W

Introduction

HILE data-driven methods for system analysis and con- trol have become increasingly popular over the recent

years, only few such methods give theoretical guarantees on, e.g., stability or constraint satisfaction of system variables [1], [2]. A control method, which is naturally well-suited for achiev- ing these objectives is model predictive control (MPC), which can handle nonlinear system dynamics, hard constraints on input, state and output, and it takes performance criteria into account [3]. It centers around the repeated online solution of an

Manuscript received November 20, 2019; revised May 4, 2020; ac-

cepted May 26, 2020. Date of publication June 5, 2020; date of cur- rent version March 29, 2021. This work was supported by Deutsche Forschungsgemeinschaft (DFG, German Research Foundation) under Germanys Excellence Strategy—EXC 2075—390740016. The authors thank the International Max Planck Research School for Intelligent Sys- tems (IMPRS-IS) for supporting Julian Berberich, and the International Research Training Group Soft Tissue Robotics (GRK 2198/1). Recom- mended by Associate Editor E. C. Kerrigan. (Corresponding author: Julian Berberich.)

Julian Berberich, Johannes Köhler, and Frank Allgöwer are with the Institute for Systems Theory and Automatic Control, University of Stuttgart 70550, Stuttgart, Germany (e-mail: julian.berberich@ist. uni-stuttgart.de; johannes.koehler@ist.uni-stuttgart.de; frank.allgower@ ist.uni-stuttgart.de).

Matthias A. Müller is with the Leibniz University Hannover, Institute of Automatic Control, 30167 Hannover, Germany (e-mail: mueller@irt.uni- hannover.de).

Color versions of one or more of the figures in this article are available online at https://ieeexplore.ieee.org.

Digital Object Identifier 10.1109/TAC.2020.3000182

optimization problem over predicted future system trajectories. Thus, for the implementation of MPC, a model of the plant is required, which is usually obtained from first principles or from measured data via system identification [4]. An appealing alternative is to implement an MPC controller directly from measured data, without prior knowledge of an accurate model. In various recent works, learning-based or adaptive MPC schemes have been proposed, which improve an inaccurate initial model using online measurements [5]–[9], while giving guarantees on the resulting closed loop. Similarly, MPC based on Gaussian processes has received increasing attraction [10], but proving desirable closed-loop properties remains an open issue. A differ- ent approach, which uses linear combinations of past trajectories to predict future trajectories, has been presented in [11], but also no guarantees on, e.g., stability of the closed loop were given. The design of purely data-driven MPC approaches with guarantees on stability and constraint satisfaction thus remains an open problem.

In this article, we present a novel data-driven MPC scheme to control linear time-invariant (LTI) systems with stability and robustness guarantees for the closed loop. Our approach relies on a result from behavioral systems theory, which shows that the Hankel matrix consisting of a previously measured input–output trajectory spans the vector space of all trajectories of an LTI sys- tem, given that the input component is persistently exciting [12]. Although this result has found various applications in the field of system identification [13]–[15], it has only recently been used to develop data-driven methods for system analysis and control with theoretical guarantees. An exposition of the main result of [12] in the classical state-space control framework and an extension to certain classes of nonlinear systems are provided in [16]. Further, the result is employed in [17] to design state- and output-feedback controllers and in [18] to verify dissipa- tion inequalities from measured data, whereas [19] investigates data-driven control without requiring persistently exciting data. Moreover, the recent contributions [20]–[22] set up an MPC scheme based on [12], but no guarantees on recursive feasibility or closed-loop stability can be given, since neither terminal ingredients are included in the MPC scheme nor sufficient lower bounds on the prediction horizon are derived. In this article, we propose a related MPC scheme, which utilizes terminal equality constraints, and we provide a theoretical analysis of various desirable properties of the closed loop. To the best of our knowledge, this is the first analysis regarding recursive feasibility and stability of purely data-driven MPC. The main advantage of the proposed MPC scheme over existing adaptive

0018-9286 © 2020 IEEE. Personal use is permitted, but republication/redistribution requires IEEE permission.

See https://www.ieee.org/publications/rights/index.html for more information.

or learning-based methods such as [5]–[9] is that it requires only

an initial condition x¯ ∈ Rn as well as a state sequence {xk}N

an initially measured, persistently exciting data trajectory as well as an upper bound on the system order, but no (set-based) model description and no online estimation process. Moreover, since it relies on the data-driven system description from [12], the presented scheme is inherently an output-feedback MPC scheme

such that

k=0

xk+1 = Axk + Buk, x0 = x¯ yk = Cxk + Duk

and does not require online state measurements.

After stating the required definitions and existing results in Section II, we expand the nominal MPC scheme of [20],

[21] by terminal equality constraints in Section III. Under the assumption that the output of the plant can be measured exactly, we prove recursive feasibility, constraint satisfaction, and ex- ponential stability of the scheme. In Section IV, we propose a robust data-driven MPC scheme to account for bounded additive noise in both the initial data for prediction as well as the online measurements. Under suitable assumptions on the system and design parameters, we prove that the closed loop under applica- tion of the scheme in a multistep fashion leads to a practically exponentially stable closed loop. In Section V, we illustrate the advantages of the proposed scheme over the scheme without terminal constraints from [20]–[22] by means of a numerical example. Section VI concludes this article.

- Preliminaries

for k = 0, ...,N  1, where (A, B, C, D) is a minimal realiza- tion of G.

—

Note that we define a trajectory of an LTI system as an input– output sequence that can be produced by a minimal realization, entailing controllability and observability of the system. Ex- tending the results of this article to systems whose input–output behavior cannot be explained via a minimal realization is an interesting issue for future research. The following result lays the foundation of the present paper. It shows that a Hankel matrix, involving a single persistently exciting trajectory, spans the vector space of all system trajectories of an LTI system. The result originates from behavioral systems theory [12], but we employ the formulation in the classical state-space control framework [16].

k=0

k

k

k=0

{	}

"  L	#

" #

Theorem 1 (see [16]): Suppose ud, yd N−1 is a trajectory of an LTI system G, where ud is persistently exciting of order L + n. Then, {u¯k, y¯k}L−1 is a trajectory of G if and only if there exists α ∈ RN−L+1 such that

Let I[a,b] denote the set of integers in the interval [a, b].

For a vector x and√ a positive definite matrix P = P × ∗ 0,

H (ud)

H (yd) α =

u¯  .	(1)

y¯

we write x  P =	x×P x. Further, we denote the minimal	L

and maximal eigenvalue of P by λmin(P ) and λmax(P ), re- spectively. For two matrices P1 = P1×, P2 = P2×, we write λmin(P1, P2) = min{λmin(P1), λmin(P2)}, and similarly for λmax(P1, P2). Moreover, x 2, x 1, and x ∞ denote the Euclidean, l1-, and l∞-norm of x, respectively. If the argument is matrix-valued, then we mean the corresponding induced norm. For δ > 0, we define Bδ = {x ∈ Rn | x  2 ≤ δ}. A sequence

k=0

{xk}N −1 induces the Hankel matrix

⎡ x0	x1	...	xN−L ⎤

Recently, Theorem 1 has received increasing attention to develop data-driven controllers [17], verify dissipativity [18], or to design MPC schemes [20]–[22]. This is due to the fact that (1) provides an appealing data-driven characterization of all trajectories of the unknown LTI system, without requiring any prior identification step. In this article, we use Theorem 1 to develop a data-driven MPC scheme with provable stability guarantees despite noisy measurements. Note that, if a sequence is persistently exciting of order L, then it is also persistently

.

x

x

⎢	1	2

...	x

N −L+1⎥

exciting of order L˜ for any L˜	L. Therefore, Theorem 1 and

≤

hence all of our results hold true if n is replaced by a (potentially

⎢

.

HL(x) :=

⎣

.	. . .	.	⎥⎦

rough) upper bound.

xL−1	xL	...	xN −1

For a stacked window of the sequence, we write

x[a,b] = ⎢⎣ . ⎥⎦ .

⎡xa⎤

.

xb

We denote by x either the sequence itself or the stacked vector

Although we assume that only input–output data of the un- known system are available, we make extensive use of the fact that an input–output trajectory of length greater than or equal to n induces a unique internal state in some minimal realization of the unknown system. We employ MPC to stabilize a desired equilibrium of the system. Since a model of this system is not available, we define an equilibrium via input–output pairs.

∈

Definition 3: We say that an input–output pair (us, ys)

Rm+p is an equilibrium of an LTI system G, if the sequence

x[0,N−1] containing all of its components. We consider the

n

n

following standard definition of persistence of excitation.

n

{u¯ , y¯ }

k  k k=0

jectory of G.

with (u¯k, y¯k) = (us, ys) for all k ∈ I[0,n] is a tra-

k=0

Definition 1: We say that a sequence {uk}N −1 with uk ∈

For an equilibrium (us, ys), we define us and ys as the

k

k

k=0

{	}

≥	∈	×

∈	⊆	∈	⊆

Rm is persistently exciting of order L if rank(HL(u)) = mL. Our goal is to control an unknown LTI system, denoted by G,

of order n with m inputs and p outputs, using only measured input–output data.

Definition 2: We  say  that  an  input–output  sequence

k=0

{uk, yk}N −1 is a trajectory of an LTI system G, if there exists

column vectors containing n times us and ys, respectively. We assume that the system is subject to pointwise-in-time input and output constraints, i.e., ut  U  Rm, yt  Y  Rp for all t  0, and we assume (us, ys)  int(U  Y). Throughout this article, ud, yd N−1 denotes an a priori measured data trajectory of length N , which is used for prediction as in (1).

The predicted input- and output-trajectories at time t over some

prediction horizon L are written as {u¯k(t), y¯k(t)}L−1 . Note

Algorithm 1: Data-Driven MPC Scheme.

that the time indices start at k = −n

k=−n inputs

- At time t, take the past n measurements u[t−n,t−1],

, since the last n

and outputs will be used to invoke a unique initial state at time

t. Further, the closed-loop input, the state in some minimal realization, and the output at time t are denoted by ut, xt, and yt, respectively.

- Nominal Data-Driven MPC

In this section, we propose a simple, nominal data-driven MPC scheme with terminal equality constraints. The scheme relies on noise-free measurements to predict future trajectories using Theorem 1 and is described in Section III-A. Under mild assumptions, we prove recursive feasibility, constraint satisfac- tion, and exponential stability of the closed loop in Section III-B.

A. Nominal MPC Scheme

Commonly, MPC relies on a model of the plant to predict fu- ture trajectories and to optimize over them. Theorem 1 provides an appealing alternative to a model since (1) suffices to capture all system trajectories. Thus, to implement a data-driven MPC scheme, one can simply replace the system dynamics constraint by the constraint that the predicted input–output trajectories satisfy (1). To be more precise, the proposed data-driven MPC scheme minimizes, at time t, given the last n input–output pairs,

y[t−n,t−1] and solve (3).

- Apply the input ut = u¯∗0(t).

- Set t = t +1 and go back to 1).

input and output. It is well-known that MPC without terminal constraints requires a sufficiently long prediction horizon to ensure stability and constraint satisfaction [23], [24]. Without such an assumption, the application of MPC can even destabilize an open-loop stable system. There are two main approaches in the literature to guarantee stability: 1) providing bounds on the minimal required prediction horizon [24] and 2) including terminal ingredients such as terminal cost functions or terminal region constraints [25]. Both approaches are usually based on model knowledge and thus, it is not straightforward to use them in the present, purely data-driven setting.

In this article, we consider a simple terminal equality con- straint, which can be directly included into the data-driven MPC framework, and which guarantees exponential stability of the closed loop. To this end, we propose the following data-driven MPC scheme with a terminal equality constraint.

JL∗ (u[t−n,t−1], y[t−n,t−1])

Σ

L−1

the following open-loop cost:

Σ

L−1

=  min

α(t)

u¯(t),y¯(t)

l (u¯k(t), y¯k(t))	(3a)

k=0

JL(u[t−n,t−1], y[t−n,t−1], α(t)) =	l (u¯k(t), y¯k(t))	(2a)

k=0

s.t.

"u¯

[−n,L−1]

(t)# =

"HL+n

(ud)#

α(t)	(3b)

"u¯[−n,L−1](t)#

"HL+n(ud)#

y¯[−n,L−1](t)

HL+n(yd)

y¯[−n,L−1]

(t)

=

HL+n

(yd)

α(t)	(2b)

"u¯

[−n,−1]

(t)# =

"u[t−n,t−1]#

y

(3c)

"u¯

[−n,−1]

(t)#

"u[t−n,t−1]#

y¯[−n,−1](t)

y[t−n,t−1]

=

y¯[−n,−1]

(t)

=

y[t−n,t−1]

.	(2c)

"u¯

n

[L−n,L−1]

(t)#

"us #

n

(3d)

As described above, the constraint (2b) replaces the system dynamics compared to classical model-based MPC schemes. Further, (2c) ensures that the internal state of the true trajectory aligns with the internal state of the predicted trajectory at time

—

—

k= n

{	}

t. Note that the overall length of the trajectory (u¯(t), y¯(t)) is L + n since the past n elements u¯k(t), y¯k(t) −1  are used to specify the initial conditions in (2c). These initial conditions are specified until time step t 1, since the input at time t might already influence the output at time t, in case of a feedthrough- element of the plant. The open-loop cost depends only on the

decision variable α(t), since u¯(t) and y¯(t) are fixed implicitly through the dynamic constraint (2b). Throughout this article, we consider quadratic stage costs, which penalize the distance w.r.t. a desired equilibrium (us, ys), i.e.,

y¯[L−n,L−1](t)	s

u¯k(t) ∈ U , y¯k(t) ∈ Y , k ∈ I[0,L−1].	(3e)

Q

The terminal equality constraint (3d) implies that x¯L(t), which is the internal state predicted L steps ahead corre- sponding to the predicted input–output trajectory, aligns with the steady-state xs corresponding to (us, ys), i.e., x¯L(t) = xs in any minimal realization. While Problem (3) requires that (us, ys) is an equilibrium of the unknown system in the sense of Definition 3, this requirement can be dropped when (us, ys) is replaced by an artificial equilibrium, which is also optimized online (compare [26]). The recent paper [27] extends the above MPC scheme to such a setting, thereby leading to a significantly larger region of attraction for the closed loop without requiring knowledge of a reachable equilibrium of the unknown system.

R

l(u¯, y¯) =  u¯ − us  2 +

y¯ − ys  2

As in standard MPC, (3) is solved in a receding horizon fashion, which is summarized in Algorithm 1.

>

where Q, R	0. In [20] and [21], it was suggested to directly minimize the above open-loop cost subject to constraints on

With slight abuse of notation, we will denote the open-loop cost and the optimal open-loop cost of (3) by JL(xt, α(t)) and

JL∗ (xt), respectively, where xt is the state in some minimal realization, induced by u[t−n,t−1], y[t−n,t−1].

Since x is the state of an observable (and hence detectable) minimal realization, there exists a matrix P ∗ 0 such that

P

W (x) =  x  2 is an input-output-to-state stability (IOSS) Lya-

B. Closed-Loop Guarantees	punov function,2 which satisfies

Without loss of generality, we assume for the analysis that

us = 0, ys = 0, and thus xs = 0. Further, we define the set

W (Ax + Bu) − W (x)

1

≤ − 2

x  2 + c1

u  2 + c2    2

(5)

2

2

y

∞}

2

of initial states, for which (3) is feasible, by XL = {x ∈ Rn | JL∗ (x) < . To prove exponential stability of the proposed scheme, we assume that the optimal value function of (3) is

quadratically upper bounded. This is, e.g., satisfied in the present

for all x  Rn,u  Rm,y = Cx + Du, and for suitable c1, c2 > 0 [29]. Define the candidate Lyapunov function V (x) = γW (x)+ JL∗ (x) for some γ > 0. Note that V is quadratically

∈	∈

lower bounded, i.e., V (x) ≥ γW (x) ≥ γλmin(P )  x  2 for all

linear-quadratic setting if the constraints are polytopic1 [28].	∗	2

Assumption 1: The optimal value function JL∗ (x) is quadrat-

x ∈ XL. Further, JL is quadratically upper bounded by Assump-

tion 1, i.e., J∗ (x) ≤ cu x 2 for all x ∈ XL. Hence, we have

ically upper bounded on XL, i.e., there exists cu

> 0 such that	L	2

L

u

2

L

2

J∗ (x) ≤ c  x  2 for all x ∈ X .

V (x) = JL∗ (x)+ γW (x) ≤ (cu + γλmax(P )) x  2

∈

Moreover, we assume that the input ud generating the data used for prediction is sufficiently rich in the following sense.

Assumption 2: The input ud of the data trajectory is persis-

for all x	XL, i.e., V is quadratically upper bounded. We consider now

tently exciting of order L + 2n.

Note that we assume persistence of excitation of order L +

λmin(Q, R)

γ =

max{c1, c2}

> 0.

2n, although Theorem 1 requires only an order of L + n. This is due to the fact that the reconstructed trajectories in (3) are

of length L + n (compared to length L in Theorem 1), since n

Along the closed-loop trajectories, using both (4) as well as (5), it holds that

components are used to fix the initial conditions. Furthermore, due to the terminal constraints (3d), the prediction horizon needs

V (x

t+1

1

2	2

) − V (xt) ≤ γ  − 2

xt  2 + c1  ut

2

2 + c2

2

yt  2

2

to be at least as long as the system order n.

− ut   − yt

γ

2

≥

Assumption 3: The prediction horizon satisfies L	n.  The following result shows that the MPC scheme based on (3)

is recursively feasible, ensures constraint satisfaction, and leads

≤ −

R	Q

.

2

xt  2

to an exponentially stable closed loop.

Theorem 2: Suppose Assumptions 1, 2, and 3 are satisfied.

If the MPC problem (3) is feasible at initial time t = 0, then

∈

it is feasible at any t  N;

∈

the closed loop satisfies the constraints, i.e., ut  U and

∈	∈

yt  Y for all t  N;

- the equilibrium xs = 0 is exponentially stable for the resulting closed loop.

Proof: Recursive feasibility (i) and constraint satisfaction

- follow from standard MPC arguments, i.e., by defining a candidate solution as the shifted, previously optimal solution and appending zero (compare [3]).

- Exponential stability

Denote the standard candidate solution mentioned above by

u¯′(t + 1), y¯′(t + 1), α′(t + 1). The cost of this solution is

It follows from standard Lyapunov arguments with Lyapunov function V that the equilibrium xs = 0 is exponentially stable with region of attraction XL.	□

The proof of Theorem 2 applies standard arguments from

model-based MPC with terminal constraints (compare [3]) to the data-driven system description derived in [12], similar to the approaches of [20] and [21], which did however not address closed-loop guarantees. To handle the fact that the stage cost l is merely positive semidefinite in the state, detectability of the stage cost is exploited via an IOSS Lyapunov function [29], similar to [30]. As we will see in Section IV, this analogy between model-based MPC and the proposed data-driven MPC scheme is only present in the nominal case, where the data is noise-free. For the more realistic case of noisy output measurements, we develop a robust data-driven MPC scheme and we provide a

JL(x

t+1

, α′(t + 1))

novel theoretical analysis of the closed loop in Section IV, which is the main contribution of this article.

L−1

L−1

Remark 1: We would like to emphasize the simplicity of the

= Σ l (u¯′k(t + 1), y¯k′ (t + 1)) = Σ l (u¯∗k(t), y¯k∗ (t))

proposed MPC scheme. Without any prior identification step,

k=0

= JL∗ (xt) − l (u¯∗0(t), y¯0∗ (t)) .

Hence, it holds that

k=1

a single measured data trajectory can be used directly to set up an MPC scheme for a linear system. Compared to other learning-based MPC approaches such as [5]–[9], which require initial model knowledge as well as an online estimation process,

JL∗ (xt+1) ≤ JL∗ (xt) − l (u¯∗0(t), y¯0∗ (t)) .	(4)

1While [28] considered model-based linear-quadratic MPC, the result applies

similarly to the present data-driven MPC setting since (3b) [together with the initial conditions (3c)] describes the input–output behavior of the system exactly and thus, both settings are equivalent in the nominal case.

the complexity of (3) is similar to classical MPC schemes, which

2Note that, in [29, Sec. 3.2] only strictly proper systems with y = Cx are considered, while we allow for more general systems with y = Cx + Du. The result from [29] can be extended to y = Cx + Du by considering a modified B˜ = B + LD in [29, Inequality (12)].

−	−

k

∈

rely on full model knowledge. To be more precise, the decision variables u¯(t), y¯(t) can be replaced by α(t) via (3b) (using a condensed formulation) and hence, since α(t) RN−L−n+1, Problem (3) contains in total N L n +1 decision variables. For ud to be persistently exciting of order L + 2n, it needs to hold that N − L − 2n +1 ≥ m(L + 2n). Assuming equality,

with a robust data-driven MPC scheme with terminal constraints. We consider output measurements with bounded additive noise in the initially available data y˜d = yd + εd as well as in the online measurements y˜k = yk + εk. We make no assumptions on the nature of the noise, but we require that it is bounded as  d  ∞ ≤ ε¯ and  εk  ∞ ≤ ε¯ for some ε¯ > 0. Thus, the present

ε

k

k

k

Problem (3) hence has m(L + 2n)+ n free parameters. On the contrary, a condensed model-based MPC optimization prob- lem contains mL decision variables for the input trajectory (assuming that state measurements are available). Thus, the online complexity of the proposed data-driven MPC approach is slightly larger (2mn + n additional decision variables) than that of model-based MPC, but it does not require an a priori (offline) identification step. It is worth noting that the difference in com- plexity is independent of the horizon L. Moreover, the proposed data-driven MPC is inherently an output-feedback controller since no state measurements are required for its implementa- tion. Finally, as in model-based MPC, for convex polytopic (or quadratic) constraints U , Y, (3) is a convex (quadratically

constrained) quadratic program which can be solved efficiently.

setting includes two types of noise. The data used for the predic- tion via the Hankel matrices in (1) is perturbed by εd, which can thus be interpreted as a multiplicative model uncertainty. On the other hand, ε perturbs the online measurements and hence, the overall control goal is a noisy output-feedback problem.

The key idea to account for noisy measurements is to relax the equality constraint (3b), where the relaxation parameter is penalized appropriately in the cost function. Given a noisy initial input–output trajectory (u[t−n,t−1], y˜[t−n,t−1]) of length n, and noisy data (ud, y˜d), we propose the following robust modification of (3):

JL∗ u[t−n,t−1], y˜[t−n,t−1]

L−1

IV.  Robust Data-Driven MPC

α(t),σ(t)

u¯(t),y¯(t) k=0

2

2

=  min  Σ l (u¯k(t), y¯k(t)) + λαε¯α(t)  2 + λσ  σ(t)  2

"

#

"

#

In this section, we propose a multistep robust data-driven MPC scheme and we prove practical exponential stability of the closed loop in the presence of bounded additive output

s.t.

u¯(t)

=

y¯(t)+ σ(t)

HL+n HL+n

ud

y˜d

α(t)

(6a)

measurement noise. The scheme includes a slack variable, which is regularized in the cost and compensates noise both in the initial

"u¯

[−n,−1]

(t)# =

"u[t−n,t−1]#

(6b)

data (ud, yd) used for prediction and in the online measure-

y¯[−n,−1](t)

y˜[t−n,t−1]

ment updates (u

[t−n,t−1]

, y[t−n,t−1]

). Section IV-A contains the

"u¯

(t)#

"us #

scheme, which is essentially a robust modification of the nominal

scheme of Section III, as well as detailed explanations of the key ingredients. In Sections IV-B and IV-C, we prove two technical

[L−n,L−1]	=

y¯[L−n,L−1](t)

n	u¯k(t)	U	(6c)

y

,

s

∈

n

Lemmas, which will be required for our main theoretical results. Recursive feasibility of the closed loop is proven in Section IV-D. In Section IV-E, we show that, under suitable assumptions, the closed loop resulting from the application of the multistep MPC scheme leads to a practically exponentially stable closed loop. Moreover, if the noise bound tends to zero, then the region of attraction of the closed loop approaches the set of all initially feasible points. In this section, we do not consider output con- straints, i.e., Y = Rp. In [31], we recently extended the results of this section by incorporating tightened output constraints in order to guarantee closed-loop constraint satisfaction despite noisy data.

- Robust MPC Scheme

In practice, the output of the unknown LTI system G is usually not available exactly, but might be subject to measurement noise. This implies that the stacked data-dependent Hankel matrices in (1) do not span the system’s trajectory space exactly and thus, the output trajectories cannot be predicted accurately. Moreover, noisy output measurements enter the initial conditions in Prob- lem (3), which deteriorates the prediction accuracy even further. Therefore, a direct application of the MPC scheme of Section III may lead to feasibility issues or it may render the closed loop un- stable. In this section, we tackle the issue of noisy measurements

σk(t) ∞ ≤ ε¯(1 + α(t) 1) , k ∈ I[0,L−1].  (6d)

Compared to the nominal MPC problem (3), the output data trajectory y˜d as well as the initial output y˜[t−n,t−1], which is obtained via online measurements, have been replaced by their noisy counterparts. Further, the following ingredients have been added.

  - A slack variable σ, bounded by (6d), to account for the noisy online measurements y˜[t−n,t−1] and for the noisy data y˜d used for prediction, which can be interpreted as a multiplicative model uncertainty,

  - Quadratic regularization (i.e., ridge regularization) of α and σ with weights λαε¯, λσ > 0, i.e., the regularization of α depends on the noise level.

2

2

2

The above l2-norm regularization for α(t) implies that small values of α(t)  2 are preferred. Since the noisy Hankel ma- trix HL+n(y˜d) is multiplied by α(t) in (6a), this implicitly reduces the influence of the noise on the prediction accuracy. Intuitively, for increasing λα, the term λαε¯α(t)  2 reduces the “complexity” of the data-driven system description (6a), similar to regularization methods in linear regression, thus allowing for a tradeoff between tracking performance and the avoidance of overfitting. The term λσ  σ(t)  2 yields small values for the slack variable σ(t), thus improving the prediction accuracy. For our theoretical results, λσ can be chosen to be zero since σ(t)

Algorithm 2: n-Step Data-Driven MPC Scheme.

- At time t, take the past n measurements u[t−n,t−1], y˜[t−n,t−1] and solve (6).

—

Apply the input sequence u[t,t+n−1] = u¯[∗0,n 1](t)

over the next n time steps.

- Set t = t + n and go back to 1).

is already rendered small by the constraint (6d). However, as we discuss in more detail in Remark 3, the constraint (6d) is nonconvex but can be neglected if λσ is large enough.

An alternative to the present regularization terms are general

Remark 3: If the constraint (6d) is neglected and the input constraint set U is a convex polytope, then Problem (6) is a strictly convex quadratic program and can be solved efficiently. However, the constraint on the slack variable σ in (6d) is non- convex due to the dependence of the right-hand side on α(t)  1, making it difficult to implement (6) in an efficient way. As will become clear later in this section, (6d) is required to prove recursive feasibility and practical exponential stability. It may,

≤

however, be replaced by the (convex) constraint  σk(t)  ∞

·

c ε¯ for a sufficiently large constant c > 0, retaining the same

theoretical guarantees. Generally, a larger choice of c increases the region of attraction, but also the size of the exponentially stable set to which the closed loop converges. Furthermore, the

P

quadratic regularization kernels, i.e., costs of the form α(t) 2 ,

α

constraint (6d) can be enforced implicitly by choosing λσ

large

P

σ(t) 2

σ

for suitable matrices Pα, Pσ ∗ 0. Further, in [21] and

enough. In simulation examples, it was observed that the con-

∞

[22], l1-regularizations of α and σ were suggested and the resulting MPC scheme, without terminal equality constraints, was successfully applied to a nonlinear stochastic control prob- lem. However, theoretical guarantees on closed-loop stability were not given. Throughout this article, we consider simple quadratic penalty terms since this simplifies the arguments, but we conjecture that our theoretical results remain to hold for general norms α(t)  p, σ(t)  q with arbitrary p, q = 1, ..., . An interesting open question, which is beyond the scope of this article, is to investigate the impact of particular choices of reg- ularization norms on the practical performance of the presented MPC approach. The choice of norms in the constraint (6d) is independent of the norms in the cost and essentially follows from

straint (6d) is usually satisfied (for suitably large choices of λσ) without enforcing it explicitly in the optimization problem and thus, it may in most cases be neglected in the online optimization. As in the previous section, we require that the measured input ud is persistently exciting of order L + 2n (Assumption 2). Further, to establish a local upper bound on the optimal cost of (6) and to prove recursive feasibility, we require that the horizon L is not shorter than twice the system’s order, as captured in the

following assumption.

≥

Assumption 4: The prediction horizon satisfies L  2n.

In some minimal realization, we denote the state trajectory corresponding to (ud, yd) by xd. According to [12, Corollary 2], Assumption 2 implies that the matrix

bound (Lemma 1) and recursive feasibility (Proposition 1).

In this section, we study the closed loop resulting from an application of (6) in an n-step MPC scheme (compare [32],

Hux =

(7)

the l∞-noise bound and the proofs of the value function upper

"	H L+n ud   #

H1

xd

[0,N −L−n]

[33]). To be more precise, we consider the scenario that, after solving (6) online, the first n computed inputs are applied to the

has full row rank and thus admits a right-inverse Hu† x =

2

Hu×x(HuxHu×x)−1. Define the quantity

system. Thereafter, the horizon is shifted by n steps, before the whole scheme is repeated (compare Algorithm 2).

cpe

:= ¨Hu† x

¨2 .	(8)

As we will see in the remainder of this section, for the considered setting with output measurement noise, the multistep MPC scheme described in Algorithm 2 has superior theoreti- cal properties compared to its corresponding one-step version. This is mainly due to the terminal equality constraints (6c),

For our stability results, we will require that cpeε¯ is bounded from above by a sufficiently small number. Essentially, this cor- responds to a quantitative “persistence-of-excitation-to-noise”- bound. To be more precise, abbreviate in the following U = HL+n(ud) and suppose that:

which complicate the proof of recursive feasibility, similar as in model-based robust MPC with terminal equality constraints

ρIm(L+n)

≤ UU × ≤ νIm(L+n)

(9)

and model mismatch. In particular, we show in this section that,

for an n-step MPC scheme with a terminal equality constraint,

for scalar constants ρ, ν > 0. Further, define the quantity cu =

pe

U †  2 =  U ×(UU ×)−1  2. Then, it holds that

2

practical exponential stability can be proven. On the other hand,

we comment on the differences for the corresponding one-step MPC scheme in Section IV-D (Remark 4). In particular, for a one-step MPC scheme relying on (6), recursive feasibility holds only locally around (us, ys) and thus, only local stability can

2

pe	2 ¨	¨2

¨	¨ ¨	¨

cu ≤ U × 2  UU × −1 2

= λmax(UU ×) · λmax  (UU ×)−1(UU ×)−1

×

be guaranteed. Nevertheless, as we will see in Section V for

λmax(UU

≤

) (9)  ν

(10)

pe

c2

a numerical example, the practical performance of the

n-step

λmin(UU ×)2

≤ ρ2 .

scheme is almost indistinguishable from the one-step scheme.

Remark 2: In the nominal case of Section III, i.e., for ε¯ = 0, (6d) implies σ = 0. Further, the regularization of α vanishes for ε¯ = 0, and the system dynamics (6a) as well as the initial conditions (6b) approach their nominal counterparts. Thus, for ε¯ = 0, Problem (6) reduces to the nominal Problem (3).

Thus, if a persistently exciting input ud is multiplied by a constant c > 1, then cu decreases proportionally to  1 . Further, the constant ρ can typically be chosen larger if the data length N increases. The same arguments can be carried out when assuming a bound of the form (9) for the matrix (7), but finding a suitable input that generates data achieving such a bound is less

obvious. It is well-known for classical definitions of persistence of excitation that larger excitation of the input implies larger excitation of the state. Therefore, we conjecture (and we have observed for various practical simulation examples) that cpe de- creases with increasing data horizons N and with multiplications

brings the state x in some minimal realization (and thus the output y) to zero in L steps. Obviously, we have u¯[−n,−1](t) = u[t−n,t−1] as well as y¯[−n,−1](t) = y˜[t−n,t−1] by (6b). By as- sumption, we have L  2n as well as 0  int(U ). Thus,

≥	∈

by controllability, there exists a δ > 0 such that for any xt

of a persistently exciting input data trajectory ud by a scalar

with  1

Γ

x

xt  2 ≤ ξt  2 ≤ δ, there exists an input trajectory

constant greater than one. This means that, for a given noise level ε¯, robust stability as guaranteed in the following sections can be obtained by choosing a large enough persistently exciting input ud and/or a sufficiently large data horizon N .

Similar to Section III, we denote the open-loop cost of the ro-

u[t,t+L−1] ∈ U L, which brings the state x[t,t+L−1] and the cor- responding output y[t,t+L−1] to the origin in L n steps while satisfying

—

2

¨

[t,t+L−1]

¨

≤ Γuy xt 2

(12)

¨"u	#¨2

bust MPC problem (6) by

J

L

and the optimal cost by ∗

[t−n,t−1]

[t−n,t−1]

¨ y[t,t+L−1] ¨2

/

(u

, y˜

, α(t), σ(t)),

JL(u[t−n,t−1], y˜[t−n,t−1]). Moreover, we assume for the analysis that (us, ys) = (0, 0). For the presented robust data-driven MPC scheme, setpoints (us, ys) = (0, 0)

change mainly one quantitative constant in Lemma 1. We com- ment on the main differences in the case (us, ys) /= (0, 0) in

for a suitable constant Γuy > 0. As candidate input–output trajectories for (6), we choose these u, y, i.e., u¯[0,L−1](t) = u[t,t+L−1], y¯[0,L−1](t) = y[t,t+L−1]. Moreover, α(t) is chosen as

Section IV-D (Remark 5).

- Local Upper Bound of Lyapunov Function

α(t) = Hu† x

u[t−n,t+L−1] xt−n

(13)

"	#

In this section, we show that the optimal cost of (6) ad- mits a quadratic upper bound, similar to the nominal case (cf. Assumption 1). It is straightforward to see that such an upper bound can not be quadratic in the state x of some minimal

realization: the optimal cost JL∗ depends explicitly on α∗(t) via

outputs

2

(u[t−n,t−1], y[t−n,t−1])

through (6a) and (6b). Even if

λαε¯α∗(t)  2, which in turn depends on the past n inputs and

where Hux is defined in (7). As is described in more detail in [15] and [16], the output of an LTI system is a linear combination of its initial condition and the input, and therefore, the above choice of α(t) implies

y[t−n,t+L−1]

α(t) =

"HL+n  ud #	"u¯[−n,L−1](t)#

HL+n

yd

arbitrarily large and hence, α and therefore also J∗

may be

the current state is zero, i.e., xt = 0, these may in general be

⎡	u¯[−n,L−1](t)	⎤

L

arbitrarily large. Thus, J∗

= ⎢⎣y¯[−n,−1](t) − ε[t−n,t−1]⎥⎦

L does not admit an upper bound in the state xt of a minimal realization. To overcome this issue, we

"	#

consider a different (not minimal) state of the system, defined as follows:

y¯[0,L−1](t)

where ε[t−n,t−1] is the true noise instance. For the slack variable

(t) = Hn ε

σ, we choose

t

ξ :=  u[t−n,t−1]  .

y[t−n,t−1]

σ[−n,−1]

d

[n,N

[0,N −L−1]

α(t) − ε

[t−n,t−1]

(14)

Further, we define the noisy version of ξ as

σ[0,L−1](t) = HL  εd

−1]  α(t)

ξ˜ := "u[t−n,t−1]# = "	u[t−n,t−1]	# .

which implies that (6a)–(6c) are satisfied. Finally, writing ei for

t

y˜[t−n,t−1]

y[t−n,t−1]

+ ε[t−n,t−1]

a row vector whose ith component is equal to 1 and which is zero otherwise, we obtain

Denote the (not invertible) linear transformation from ξ to

an arbitrary but fixed state x in some minimal realization by

2	2

t

t

t

2

2

t

2

T , i.e., x = Tξ . Clearly, this implies  x  ≤ T  ξ 2 =:

HL+n(εd)α(t) 	=	max

∞

i∈I[1,p(L+n)]

|eiHL+n(ε )α(t)|

d

(15)

2

P

Γx  ξt  2. Note that ξ is the state of a detectable state-space realization and thus, there exists an IOSS Lyapunov function W (ξ) =  ξ  2 , similar to the proof of Theorem 2. For some γ > 0, define Vt := JL∗ (ξ˜t)+ γW (ξt). The following result shows that, for the state ξ, a meaningful quadratic upper bound on V can be proven.

Lemma 1: Suppose Assumptions 2 and 4 hold. Then, there exists a constant c3 > 0 as well as a δ > 0 such that, for all

≤ ε¯ α(t) 1.

≤

This implies  σ(t)  ∞	ε¯(  α(t)  1 + 1), which in turn proves that (6d) is satisfied.

In the following, we employ the above candidate solution to bound the optimal cost and thereby, the function V . Due to observability of the pair (A, C), corresponding to the minimal realization with state x, it holds that

"u¯	(t)#	"I	0 #

ξt ∈ Bδ, Problem (6) is feasible and V is bounded as

γλmin(P )  ξt  2 ≤ Vt ≤ c3  ξt  2 + c4	(11)

[−n,−1]

`	˛¸	x

xt−n

=	mn

M1	Φ†

ξt	(16)

2	2

where c4 = 2npε¯2λσ.

Proof: The lower bound is trivial. For the upper bound, we construct a feasible candidate solution to Problem (6) which

M :=

where Φ† = (Φ×Φ)−1Φ× is a left-inverse of the observability matrix Φ. The lower block of (16) follows from observability and

∈

2

k	2

2 +  xt−n

2

2

¨ ⎠

the linear system dynamics xk+1 = Axk + Buk, yk = Cxk + Duk for k  I[t−n,t−1], which can be used to compute the matrix M1 depending on A, B, C, D. Hence, α(t) can be bounded as

which will play an important role in proving recursive feasibility and practical stabiliy of the proposed scheme. For an integer k, define constants ρ2,k, ρ∞,k such that

(13)

α(t) 2

≤

Hux

2	¨

† ¨2 ¨

¨		2

ρ2,k ≥ ¨CA Φ†¨

2

2

u¯[−n,L−1](t)

⎛

= ¨Hu† x

¨2 ⎝¨u¯

∞

[0,L−1]

(t)¨2 + ¨

u¯[−n,−1](t)

where Φ†

is a left-inverse of the observability matrix Φ.

(12),(16)

¨	xt−n	¨2

Lemma 2: If (6) is feasible at time t, then the following

inequalities hold for all k ∈ I[0,L−1]

2

¨"

#¨2⎞

ρ∞,k ≥ ¨CAkΦ†¨

≤	¨H†  ¨2 Γ

x 2 + M 2

ξ 2 .	(17)

ux

2

`  ˛¸  x

uy	t 2	2

t 2	yˆt+k − y¯k∗ (t)  2 ≤ 8c5ε¯2  α∗(t)  2 +2  σk∗ (t)  2

(20)

Using standard norm equivalence properties, it holds for arbi-

cpe=

2

+ ρ2,n+k

16nε¯

c5 α (t) 2 + p

+4 σ[−n,−1](t) 2

2	 ∗

2	2

2	 ∗	 2

trary k ∈ N that

yˆt+k − y¯k∗ (t)  ∞ ≤ ε¯α∗(t)  1 +  σk∗ (t)  ∞

1	¨ [ n, 1]	¨

¨

k

[0,N −L−n+k−1]

¨2

5

+ ρ∞,n+k

ε¯( α∗(t)	+ 1) +  σ∗	(t)

− −

∞

(21)

¨H  εd	 ¨2 ≤ c kε¯2	(18)			¨	¨

≤

−	−

where c5 := p(N	L	n + 1). Based on the definition of σ(t) in (14), and using (18) as well as the inequality (a + b)2 2(a2 + b2), we can bound σ(t) in terms of α(t) as

σ(t)  2 ≤ 2npε¯2 + c (L + 2n)ε¯2  α(t)  2.	(19)

with c5 from (18).

Proof: We show only (21) and note that (20) can be de- rived following the same steps, using (18) as well as the in-

equality (a + b)2 ≤ 2a2 + 2b2. As written above, yˆ is the tra-

2	5	2

Combining the above inequalities, V is upper bounded as

Vt ≤ JL(ξ˜t, α(t), σ(t)) + γW (ξt)

jectory, resulting from an open-loop application of u¯∗(t) and with initial conditions specified by (u[t−n,t−1], yˆ[t−n,t−1]) = (u[t−n,t−1], y[t−n,t−1]). On the other hand, according to (6a), y¯∗(t) is comprised as

≤ λmax(Q, R)Γuy  xt

2 + γλmax(P )  ξt  2

y¯∗(t) = HL+n

εd α∗(t)+ H

L+n

yd α∗(t) − σ∗(t).

2

2

+ (λα + c5(L + 2n)λσε¯) cpeε¯ Γuy  xt  2 +  M 2

ξt  2

It follows directly from (6a) and (6b) that the second term on the

2	2	2

+ 2npε¯2λσ.

2

2

Finally, xt is bounded by ξt as  xt  2 ≤ Γx  ξt  2, which leads

right-hand side HL+n(yd)α∗(t) is a trajectory of G, resulting from an open-loop application of u¯∗(t) and with initial output

conditions

2

[t−n,t−1]

[−n,−1]

n

[0,N −L−1]

to Vt ≤ c3  ξt  2 + c4, where

c

3

= λ

max

(Q, R)Γuy

Γ

x

+ γλ

max

(P )

y˜	+ σ∗	(t) − H  εd	 α∗(t).

+ (λα

+ c5(L + 2n)λσ

ε¯) cpe

ε¯ ΓuyΓx

+  M 2

Define

y−

= yˆ[t−n,t+L−1] − HL+n yd α∗(t).

2

c4 = 2npε¯2λσ.

[t−n,t+L−1]

□	Since G is LTI and y− contains the difference between two

In Section III, we assumed that the optimal cost is quadrati- cally upper bounded (cf. Assumption 1), which is not restrictive in the nominal linear-quadratic setting. Lemma 1 proves that, under mild assumptions, the optimal cost of the robust MPC problem (6) admits (locally) a similar upper bound and can thus be seen as the robust counterpart of Assumption 1.

The term c4 is solely due to the slack variable σ. This

trajectories with the same input, we can assume u¯∗(t) = 0 for the following arguments without loss of generality. Hence, y− is equal to the output component of a trajectory (u−, y−) with zero input and with initial trajectory

"

"	#

u[−t−n,t−1] y[−t−n,t−1]

—

can be explained by noting that, for ξt = 0, α(t), u¯[0,L−1](t), y¯[0,L−1](t) can all be chosen to be zero, as long as σ compensates the noise, i.e., σ[−n,−1](t) = −ε[t−n,t−1].

=  H	εd

n	[0,N −L−1]

0

α∗(t) − ε

[t−n,t−1]

— σ[∗ n,−1]

(t)# .

- Prediction Error Bound

—

—

The relation to the internal state x− can be derived as

(22)

Denote the optimizers of (6) by α∗(t), σ∗(t), u¯∗(t), y¯∗(t), and the output trajectory resulting from an open-loop application of

y[−t n,t−1]

= Φxt− n

u¯∗(t) by yˆ. One of the reasons why it is difficult to analyze the presented MPC scheme is the nontrivial relation between the predicted output y¯∗(t) and the “actual” output yˆ. In the following,

with the observability matrix Φ. This leads to the corresponding output at time t + k

y−	= CAn+kΦ y−

we derive a bound on the difference between the two quantities,

t+k

† [t−n,t−1]

where Φ† is a left-inverse of Φ. Using this fact, the expression

Hence, if ε¯0 is sufficiently small, then yˆt+k becomes arbi-

for y[−t

n,t−

in (22), and the inequality (15),  yt−+k  ∞ can be

trarily small at the above time instants. This implies that the

Φ†yˆ[t+L−n,t+L−1], approaches zero for ε¯	0. Thus, similar to the proof of Lemma 1, there exists an input trajectory

bound−ed as1]

internal state in some minimal realization corresponding to

t+k  ∞

y− 	≤ ρ

ε¯(  α∗(t)

+ 1) +  σ∗

(t)	 .

the trajectory (u¯∗(t), yˆ) at time t + L − n, i.e.,

→

xˆt+L−n =

∞,n+k

1

[−n,−1]

∞

Note that

u¯′[L−2n,L−n−1]

(t + n), which brings the state and the corre-

−	− −

′

¨

¨

yˆt+k − y¯k∗ (t)  ∞ ≤ yt−+k  ∞ + ε¯ α∗(t)  1 +  σk∗ (t)  ∞

which concludes the proof.	□

Essentially, Lemma 2 gives a bound on the mismatch between

sponding output y¯[L 2n,L n 1](t + n) to zero in n steps, while satisfying

¨"u¯′	(t + n)#¨2

the predicted output and the actual output resulting from the

open-loop application of u¯∗(t), depending on the optimal solu-

[L−2n,L−n−1]

2

¨ y¯[′L−2n,L−n−1]

(t + n)

≤ Γuy

¨

2

xˆt+L−n  2.	(23)

−	−

tions α∗, σ∗, and on system parameters. In model-based robust MPC schemes, similar bounds are typically used to propagate uncertainty, where the role of the weighting vector α to account

Moreover, in the interval I[L−n,L−1], we choose u¯′[L−n,L−1](t + n) = 0, y¯[′L n,L 1](t + n) = 0, i.e., (6c) is satisfied. The above arguments imply that

for multiplicative uncertainty is replaced by the state x and a model-based uncertainty description (compare [34] for details). The main difference in the proposed MPC scheme is that the

u¯′(t + n),

yˆ[t,t+n−1] y¯[′0,L−1](t + n)

"

#!

predicted trajectory y¯∗(t) is in general not a trajectory of the system in the sense of Definition 2, corresponding to the input

u¯∗(t). On the contrary, in model-based robust MPC, the pre- dicted trajectory usually satisfies the dynamics of a (nominal) model of the system.

is a trajectory of the unknown LTI system in the sense of Definition 2. Denote the corresponding internal state in some minimal realization by x¯′(t + n). We choose α′(t + n) as a corresponding solution to (1), i.e., as

- Recursive Feasibility

α′(t + n) = Hu† x

u¯′[−n,L−1](t + n)	(24)

"	#

xt

The following result shows that, if the proposed robust MPC scheme is feasible at time t, then it is also feasible at time t + n, assuming that the noise level is sufficiently small.

≤	≥

≤

Proposition 1: Suppose Assumptions 2 and 4 hold. Then, for any VROA > 0, there exists an ε¯0 > 0 such that for all ε¯ ε¯0, if Vt  VROA for some t  0, then the optimization problem (6) is feasible at time t + n.

with Hux from (7). Finally, we fix

σ′(t + n) = HL+n y˜d α′(t + n) − y¯′(t + n)	(25)

—

[0,N

which implies that (6a) holds. It remains to show that the constraint (6d) is satisfied. Over the first n time steps, (6d) holds since

Proof: Suppose the robust MPC problem (6) is feasi-

σ[′ n,−1](t + n) = Hn y˜d

−L−1] α′(t + n) − y˜[t,t+n−1]

≤

ble at time t with Vt	VROA and denote the optimizers by

α∗(t), σ∗(t), u¯∗(t), y¯∗(t). As in Lemma 2, the trajectory result-

ing from an open-loop application of u¯∗(t) and with initial con-

(24)

= Hn

εd

[0,N −L−1]

α′(t + n)+ y[t,t+n−1] − y˜[t,t+n−1]

ditions specified by (u[t−n,t−1], y[t−n,t−1]) is denoted by yˆ. For

= Hn  εd

α′(t + n) − ε[t,t+n−1].	(26)

k ∈ I

, we choose for the candidate input the shifted

[0,N −L−1]

[−n,L−2n−1]

′	∗	. Over the

Further, using the definition of σ′(t + n) in (25) and the

previously optimal solution, i.e., u¯k(t + n) = u¯k+n(t)

first n steps, the candidate output must satisfy y¯[′

(t + n) =

bound (15), we obtain

∈

y˜[t,t+n−1]

due to (6b). Further, for k	I

′

−n,−1] , the out-

[0,L−2n−1]

σ[′0,L−1](t + n)  ∞ ≤ ε¯α′(t + n)  1

put is chosen as y¯k(t + n) = yˆt+n+k. Since y¯[∗L−n,L−1](t) = 0

¨				¨

¨

¨

∞

[n,N −1]

[0,L−1]

by (6c), the prediction error bound of Lemma 2 implies that, for

any k ∈ I[L−n,L−1], it holds that

+  HL  yd

α′(t + n) − y¯′

(t + n)

(27)

For

yˆt+k  ∞ ≤ ε¯α∗(t)  1 +  σ∗(t)  ∞

—

+ ρ∞,n+k ε¯(  α∗(t)  1 + 1) +  σ[∗ n,−1](t)  ∞ .

ε¯0 sufficiently small,   σ∗(t)  ∞ becomes arbitrarily

and thus, (6d) holds.	□

Proposition 1 shows that, for any sublevel set of the Lyapunov

`

=˛¸0

x

function V , there exists a sufficiently small noise bound ε¯0 such that, for any ε¯ ≤ ε¯0 and any state starting in the sublevel set

2

small due to (6d). Further, using that λαε¯α∗(t)  2 ≤

JL∗ (ut−n,t−1], y˜[t−n,t−1]) ≤ VROA, we can bound α∗(t) as

≤	−	−	r

1	N − L − n +1	2

α∗(t)   ≤ √	  α∗(t)

√N	L	n + 1	VROA .

λαε¯

at time t, the n-step MPC scheme is feasible at time t + n. In particular, the required noise bound decreases if the size of the sublevel set, i.e., VROA, increases and vice versa. This can be explained by noting that the noise in (6a) corresponds to a multiplicative uncertainty, which affects the prediction accuracy more strongly if the current state is further away from the origin and hence the Lyapunov function Vt is larger. We note that this

2

/	/

≤	−

/

/

2

2

be straightforwardly extended to the case (us, ys) = 0, with the corresponding steady state ξs = 0. The main difference lies in the bound (11), which becomes Vt c˜3  ξt ξs  2 + c˜4 for constants c˜3 = c3, c˜4 = c4, where c˜3 can be made arbitrarily close to c3. On the other hand, c˜4 changes depending on ξs, since the right-hand side of (17) would need to be proportional to ξt − ξs  2 +  ξs  2. The same phenomenon can be observed

—

Fig. 1. Sketch of the candidate output for recursive feasibility. Due to the terminal equality constraints (6c), the last n steps of the optimal predicted output y¯∗(t) are equal to zero. According to the prediction error bound derived in Lemma 2, this implies that the state resulting from an open-loop application of the optimal input u¯∗(t) is small at time L

2n, provided that ε¯ is sufficiently small. Therefore, a candidate solution

y¯′(t + n) can be constructed by appending the open-loop output yˆ by a local deadbeat controller, which steers the state to the origin in n steps.

≤

≤

does not imply recursive feasibility of the n-step MPC scheme in the standard sense since it remains to be shown that the sublevel set Vt  VROA is invariant, which will be proven in Section IV-E. In our main result, the set of initial states for which V0  VROA will play the role of the guaranteed region of attraction of the closed-loop system.

−	−

The input candidate solution used to prove recursive feasi- bility in Proposition 1 is analogous to a candidate solution one would use to show robust recursive feasibility in model-based robust MPC with terminal equality constraints. The output can- didate solution is sketched in Fig. 1. Up to time L  2n  1,

y¯′(t + n) is equal to yˆ (shifted by n times steps), which is the output, resulting from an open-loop application of u¯∗(t). This choice together with the prediction error bound of Lemma 2

—

implies that the internal state corresponding to y¯′(t + n) at time L 2n is close to zero. Thus, by controllability, there exists an input trajectory satisfying the input constraints, which

brings the state and the output to zero in n steps. In the interval I[L−2n,L−n−1], the candidate output is chosen as this trajectory. This also implies that the choice y¯[′L−n,L−1](t + n) = 0 makes the candidate solution between 0 and L − 1, i.e., (u¯′[0,L−1](t +

in a bound of α′(t + n) based on (24), which will be used in the stability proof. As will become clear later in this section, such

changes in the bound of α′(t + n) as well as in the constant

c˜4 do not affect our qualitative theoretical results, but they

/

may potentially (quantitatively) deterioriate the robustness w.r.t. the noise level ε¯. Intuitively, this can be explained by noting that (6a) corresponds to a multiplicative uncertainty and thus, stabilization of the origin is simpler than stabilization of any other equilibrium. Since equilibria with (us, ys) = 0 require a significantly more involved notation, we omit this extension.

- Practical Exponential Stability

The following is our main stability result. It shows that, under Assumptions 2 and 4, for a low noise amplitude and large persistence of excitation, and for suitable regularization parameters, the application of the scheme (6) as described in Algorithm 2 leads to a practically exponentially stable closed loop.

Theorem 3: Suppose Assumptions 2 and 4 hold. Then, for any VROA > 0, there exist constants λα, λα, λσ, λσ > 0 such that, for all λα, λσ satisfying

λα ≤ λα ≤ λα,  λσ ≤ λσ ≤ λσ	(28)

→

there exist constants ε¯0, c¯pe > 0, as well as a continuous, strictly increasing β : [0, ε¯0] [0, VROA] with β(0) = 0, such that, for all ε¯, cpe satisfying

≤

ε¯ ≤ ε¯0, cpeε¯ ≤ cpe	(29) the sublevel set V ≤ V	is invariant and V converges ex-

n), y¯′	(t + n)), a trajectory3 of the unknown system G in

t	ROA	t

—

[0,L  1]

the sense of Definition 2. Finally, the suggested candidate input

is also similar to [35], where inherent robustness of quasi-infinite horizon (model-based) MPC is shown.

Remark 4: For a one-step MPC scheme, a similar argu- ment to prove recursive feasibility can be applied, given

ponentially to Vt β(ε¯) in closed loop with the n-step MPC scheme for all initial conditions for which V0  VROA.

≤

Proof: The proof consists of three parts: First, we bound the increase in the Lyapunov function V . Thereafter, we prove that, for suitably chosen bounds on the parameters, there exists a

function β, which satisfies the above requirements. Finally, we

that

u¯[∗L−2n,L−n−1](t) and

y¯[∗L−2n,L−n−1](t) (and hence

show invariance of the sublevel set Vt ≤ VROA and exponential

≤

yˆ[t+L−2n,t+L−n−1]) are close to zero. This is required to con- struct a feasible input which steers the state and the corre-

sponding output to zero, similar to the proof of Proposition 1, and it is, e.g., the case if the initial state xt is close to zero. That is, the result of Proposition 1 holds locally for a one-step MPC scheme, as expected based on model-based MPC with terminal equality constraints under disturbances using inherent robustness properties.

Remark 5: As mentioned in Section IV-A, all of our theo- retical guarantees for the presented robust MPC scheme can

3In most practical cases, (u¯∗(t), y¯∗(t)) are not trajectories of the system due to the slack variable σ and the noise.

convergence of Vt to Vt  β(ε¯).

- Practical Stability

Suppose Problem (6) is feasible at time t and let VROA > 0 be arbitrary. Further, let ε¯0 be sufficiently small such that Proposi- tion 1 is applicable. The cost of the candidate solution derived in Proposition 1 at time t + n is

JL  u[t,t+n−1], y˜[t,t+n−1], α′(t + n), σ′(t + n)

Σ	2

L−1

k

k

α

2

=	l (u¯′ (t + n), y¯′ (t + n)) + λ ε¯α′(t + n)

k=0

2

+ λσ  σ′(t + n)  2.

Thus, we obtain for the optimal cost

≤ λmax(Q, R)Γuy  Φ†  2 8c5nε¯2  α∗(t)  2 +2  σ∗(t)  2

JL∗ (u[t,t+n−1], y˜[t,t+n−1])

′	′

L−1

+

2	2	2

Σ

ρ2,n+k

≤ JL

u[t,t+n−1], y˜[t,t+n−1],α (t + n),σ (t + n)

Σ

= J∗ (u

, y˜

) −

l (u¯∗ (t), y¯∗ (t))

L−1

k=L−n

2

[−n,−1]

· 16nε¯2 c5  α∗(t)  2 + p +4  σ∗

2

(t)  2  .	(32)

L	[t−n,t−1]

[t−n,t−1]

k	k

k=0

Next, we bound the difference between the third and the fourth term on the right-hand side of (31). The following relations are

— λαε¯ α∗(t) 2 − λσ σ∗(t) 2 + λαε¯ α′(t + n) 2

readily derived:

2	2	2

L−1

′	 2	 ∗	 2

Q

σ

2

k

k

y¯k′ (t + n) − y¯k∗+n(t)+ y¯k∗+n(t) 2

−

+ λ σ′(t + n)  2 + Σ l(u¯′ (t + n), y¯′ (t + n)).	(30)

k=0

y¯k(t + n) Q −

=

y¯k+n(t) Q

Q

y¯k∗+n(t)  2

k

k+n

Q

In the following key technical part of the proof [Parts (i.i)–(i.iv)[,

we derive useful bounds for most terms on the right-hand side

=  y¯′ (t + n) − y¯∗	(t)  2

k

k+n

k+n

of (30). This will lead to a decay bound of the optimal cost which

is then used to prove practical exponential stability of the closed

+2 y¯′ (t + n) − y¯∗	(t) × Qy¯∗

(t)

loop.

(i.i) Stage Cost Bounds

We first bound those terms in (30), which involve the stage

≤ y¯k′ (t + n) − y¯k∗+n(t)  2

+2  y¯k′ (t + n) − y¯k∗+n(t)  Q  y¯∗k+n(t)  Q.	(33)

Q

cost. The above difference can be decomposed as

By	using	2  y¯k∗+n(t)  Q ≤ 1+  y¯∗k+n

(t)  2

as	well	as

Q

Σ

L−1

l(u¯′ (t + n), y¯′ (t + n)) −

L−1

l (u¯∗ (t), y¯∗ (t))

y¯k∗+n

(t)  2

≤ VROA, we arrive at

Q

k	k

k=0

k	k

Σ

k=0

2  y¯k′ (t + n) − y¯k∗+n(t)  Q

y¯k∗+n(t)  Q

Σ

Σ

L−n−1

=

k=L−2n

l (u¯′k(t + n), y¯k′ (t + n)) −

n−1 k=0

l (u¯∗k(t), y¯k∗ (t))

≤ y¯k′ (t + n) − y¯k∗+n(t)  Q (1 + VROA) .	(34)

Σ

k

k

k+n

k+n

Therefore, since the inputs coincide over the considered time interval, and due to (33) as well as (34), it holds that

Σ

L−2n−1

+

k=0

l (u¯′ (t + n), y¯′ (t + n)) − l u¯∗

(t), y¯∗

(t)

L−2n−1

k=0

l (u¯′k(t + n), y¯k′ (t + n)) − l

u¯∗k+n(t), y¯k∗+n(t)

∈

Q

(31)

where we use that u¯′k(t + n), y¯k′ (t + n), u¯∗k(t), y¯k∗ (t) are all zero for k I[L−n,L−1] due to (6c). To bound the first term on the right-hand side of (31), note that

L−2n−1

Σ

≤

k=0

y¯k′ (t + n) − y¯k∗+n(t)  2

2

xˆ	 ≤ Φ

2  yˆ	2

+  y¯k′ (t + n) − y¯k∗+n(t)  Q (1 + VROA) .	(35)

t+L−n  2

† 2	[t+L−n,t+L−1]  2

The difference  y¯′ (t + n) − y¯∗	(t) 	can be bounded sim-

k

k+n

Q

−	−

with xˆt+L−n as in the proof of Proposition 1. Further, since y¯[∗L n,L 1](t) = 0, yˆ can be bounded in the considered time interval as in (20), i.e.,

ilar to Lemma 2. Using the constraint (6d) to bound  σ∗(t) 2, it can be shown that the bound is of the form y¯′k(t + n) − y¯∗k+n(t)  Q ≤ C˜1  α∗(t)  2 + C˜2 ≤ C˜1(1 +

yˆ	2 ≤ 8c nε¯2  α∗(t)  2 +2  σ∗(t)  2

α∗(t)  2)+ C˜2, where both C˜1 and C˜2 are proportional to ε¯.

[t+L−n,t+L−1]  2

Σ

L−1

5	2	2

2

2

2

Ci > 0, where C1 and C3 are quadratic in ε¯and vanish for ε¯ = 0. Therefore, if λα and λσ are sufficiently large, then (30) implies

Hence, applying Lemma 2 to (35), the sum of (32) and (35) can be bounded as C1  α∗(t)  2 + C2  σ∗(t)  2 + C3 for suitable

+

ρ2,n+k

k=L−n

· 16nε¯2 c5  α∗(t)  2 + p +4  σ∗

(t)  2  .

JL∗ (u[t,t+n−1], y˜[t,t+n−1])

2

L

[t−n,t−1]

Hence, it holds that

[−n,−1]	2

≤ J∗ (u

, y˜

n−1

Σ

[t−n,t−1]

k=0

k

k

) −	l (u¯∗ (t), y¯∗ (t))

L−n−1

2

k=ΣL−2n

l (u¯′k(t + n), y¯k′ (t + n))

+ λα

ε¯α′(t + n)  2 + λσ

σ′(t + n)  2 + c6

(36)

2

2

(23)

≤ λmax(Q, R)Γuy

xˆt+L−n  2

for a suitable constant c6 > 0, which is quadratic in ε¯ and vanishes for ε¯ = 0.

2

(i.ii) Bound of σ′(t + n)  2

By applying standard norm bounds to the slack variable can-

didate σ′(t + n) as defined in (25) [compare also (26) and (27)],

y[t,t+n−1]	y¯[∗0,n 1](t)  2 vanish in (40) if λσ, λα are chosen sufficiently large, except for an additive constant, which depends

2

−	−

solely on the noise. Moreover, choosing γ =  λmin(Q,R) , it

we obtain

σ′(t + n)  2 ≤ 2npε¯2 + c5(L + 2n)ε¯2  α′(t + n)  2

(37)

holds that

2	 ∗	 2

max{c1,2c2}

2	2

(i.iii) Bound of  α′(t + n)  2

with c5 = p(N − L − n + 1) as in (18).

γ(c1 u[t,t+n−1]

2 + 2c2

y¯[0,n−1](t) 2)

Σ

k

k

n−1

2

For the weighting vector α′(t + n), it holds that

≤

k=0

l(u¯∗ (t), y¯∗ (t)).

(24)

¨"u¯′

(t + n)#¨2

Combining these facts, we arrive at

2

α′(t + n)  2

≤ cpe ¨

[−n,L−1]

¨

−n

2

¨	x¯′

(t +n)	¨2

Vt+n − Vt ≤

γ

(λα + λσc7) Γxcpeε¯ − 2

ξt  2

= cpe  xt 2 +

u¯′	(t + n) 2  = cpe xt 2	 ∗	 2

2

+ cpe   u¯∗

[−n,L−1]

(t)  2 +

2

u¯′

2

(t + n)  2  .

+ (λα + λσc7) cpeε¯ u¯[0,L−n−1](t) 2 + c9

for a suitable constant c9, which vanishes for ε¯ = 0. Finally, note

[0,L−n−1]	2

[L−2n,L−n−1]	2

that λ

(R)  u¯∗

(t)  2 ≤ V , which leads to

Similar to (32), we can use (23) to bound the last term as

min

[0,L−n−1]	2	t

u¯[L−2n,L−n−1](t + n) 2

t+n

′	 2	V

— V ≤  (λ + λ c )Γ c  ε¯ − γ  ξ 2

Σ

2

t

α

σ

7

x pe

2

t

2

≤ Γuy Φ†

8c5nε¯2  α∗(t)  2 +2  σ∗(t)  2 +

L−1

+ (λα + λσc7) cpeε¯V + c

ρ2,n+k

2	2	2

λ

(R)

t

9

k=L−n

min

γ 	2

5

2

[−n,−1]

2

· 16nε¯2 c  α∗(t)  2 + p +4  σ∗	(t)  2   .	(38)

=:  c10 − 2	ξt 2 + c11Vt + c9.	(41)

The bound (38) is of the same form as (32) and (35). Due to this fact, using the bound (37) for σ′(t + n), and by potentially choosing λα and λσ larger, (30) implies

- Construction of β

∈

The local upper bound in Lemma 1, which holds for any ξt Bδ, implies that the following holds for any VROA > 0, and any ξt with Vt ≤ VROA:

JL∗ (u[t,t+n−1], y˜[t,t+n−1])

n−1

Vt ≤ max

c3,

VROA − c4

2

δ2

ξt  2 + c4.	(42)

Σ

≤ JL∗ (u[t−n,t−1], y˜[t−n,t−1]) −	l (u¯∗k(t), y¯k∗ (t))

:= γ − c

— c3c11

k=0

`	c3,VR˛O¸A :=	x

α

σ

7

t

2

[0,L−n−1]

2

pe

8

+ (λ  + λ c )   x 2 +  u¯∗	(t)  2 c  ε¯ + c	(39)

We first consider VROA = δ2c3 + c4, which implies c3,VROA = c3.

Further, we define c

12

10

as well as

for suitable constants c7, c8 > 0, which vanish for ε¯ = 0.

(i.iv) IOSS Bound

2

2

As in the proof of Theorem 2, we consider now Vt = JL∗ (ξ˜t)+ γW (ξt) with the IOSS Lyapunov function W for some γ > 0. It follows directly from (5), (39), and from xt  2 ≤ Γx  ξt  2 that

γ c4 + c3c9 β(ε¯) =  2

2

c12

4

4

9

5

6

10

7

8

11

9

for any ε¯ for which c12 > 0. Recall that c3 = a1ε¯2 + a2ε¯ + a3, c = a ε¯2, c = a ε¯2 + a ε¯, c	= a ε¯2 + a ε¯, c	= a ε¯2 +

Σ

n−1

Vt+n − Vt ≤ −	l (u¯∗k(t), y¯k∗ (t))

k=0

a10ε¯, for suitable constants ai > 0. This implies β(0) = 0. Next, we show the existence of a constant ε¯0 such that β is strictly increasing on [0, ε¯0]. If c12 > 0, then β is strictly increasing

1

2

+ γ  − 2 ξ[t,t+n−1]

2 + c1  u[t,t+n−1]

2

2

2 + c2  y[t,t+n−1]  2

since its numerator increases with ε¯, whereas its denominator decreases with ε¯. In the following, we show that c12 > 0. By

definition, we have

2

+ (λα + λσc7) Γx  ξt  2 +

u¯[∗0,L−n−1]

(t)  2 cpeε¯ + c8.

2

(40)

c12 =

γ

2 − (λα + λσc7) Γxcpeε¯

The identity (a + b)2 ≤ 2(a2 + b2) yields

(λα + λσc7) cpeε¯

—	(λ

(Q, R)Γ

Γ  + γλ

(P )

2	 ∗	 2

λmin(R)

max

uy  x	max

2

y[t,t+n−1]

2 ≤ 2 y¯[0,n−1](t) 2

+ (λα

+ c5(L + 2n)λσ

ε¯) cpe

ε¯(ΓuyΓx

+  M 2 .

2

+2  y[t,t+n−1] − y¯[∗0,n−1]

(t)  2

It can be seen directly from this expression that, if

λα ≤

where the latter term can again be bounded using Lemma 2. Sim- ilar to the earlier steps of this proof, the components of the bound

λα, λσ	λσ, with arbitrary but fixed upper bounds λα, λσ, and

≤

cpeε¯ is sufficiently small, then c12 > 0. It remains to show that

β(ε¯0) ≤ VROA, or, equivalently,

2

2

γ	≤	3	4

γ c4 + c3c9

δ c + c

2 − c10 − c3c11

which can be ensured by choosing ε¯0 sufficiently small.

- Invariance and Exponential Convergence

≤

2

Take an arbitrary ξt with Vt  VROA and note that this implies that (6) is feasible and thus, (41) and (42) hold. Moreover, c12 > 0 implies c10 < γ . Defining Vβ,t := Vt − β(ε¯), we thus obtain

are also valid without imposing the lower bound in (28) on λσ, by using the more conservative constraint (6d) in the proof. However, (6d) is nonconvex (cf. Remark 3), but can typically be enforced implicitly if λσ is chosen large enough.

≤

In the proof of Theorem 3, a close connection between the region of attraction, i.e., the set of initial conditions with V0 VROA, and various parameters becomes apparent. First of all, the noise bound ε¯ needs to be sufficiently small depending on VROA to allow for an application of Proposition 1. Moreover,

t

if VROA increases, then also c3,VROA increases and hence, c11

Vt+n

(41)

≤ (1 + c11

) Vt

+  c10

— γ  ξ

2

2 + c9

must decrease to ensure c

stability. To render

12,VROA > 0 and thereby exponential must decrease, i.e., the amount

(42)

c10 − γ

c4  γ

c11 small, cpeε¯

→

of persistence of excitation compared to the noise level must

2

2

3

≤	1+ c11 +	c

Vt +

c

3

2 − c10

+ c9

increase. Thus, for cpeε¯	0 (and a sufficiently small noise bound ε¯due to Proposition 1), the region of attraction approaches

≤	1+ c11 +

c10 − γ

Vβ,t + β(ε¯)

the set of all initially feasible points. For a fixed cpe

, the size of the

2

c3

where the last inequality follows from elementary computations. This in turn implies the following contraction property

region of attraction increases if the noise level decreases and vice versa. A similar connection between the maximal disturbance and the region of attraction can be found in [35], which studies

inherent robustness properties of quasi-infinite horizon MPC

Vβ,t+n ≤

`

1+ c11 +

<˛¸1

γ

c10 −

2	Vβ,t.	(43)

c3

x

(but the result applies similarly to model-based n-step MPC with terminal equality constraints). Further, if cpe decreases then so do c10 as well as c11 and hence also β(ε¯). This implies that

≤

≤

If the noise bound ε¯0 is sufficiently small, then this im- plies invariance of the sublevel set Vt VROA and hence, by Proposition 1, recursive feasibility of the n-step MPC scheme. Applying the contraction property (43) recursively, we can thus conclude that Vt converges exponentially to Vt  β(ε¯).

So far, we have only considered the case VROA = δ2c3 + c4. It remains to show that, for any VROA > 0, there exist suitable parameter bounds such that

larger persistence of excitation (i.e., a lower cpeε¯) does not only increase the region of attraction but it also reduces the tracking error.

Remark 6: To apply the proposed data-driven MPC scheme in practice, the following ingredients are required. First of all, the design parameters in the cost, i.e., Q, R, λα, λσ, have to be selected suitably. The proof and discussion of Theorem 3 give a qualitative guideline for choosing the regularization parameters. Further, as in the nominal case (see Section III), measured data

c12,VROA

γ

:= 2 − c10

— c3,VROA

c11 > 0

with a persistently exciting input as well as a (potentially rough) upper bound on the system’s order need to be available. Finally,

with c3,VROA from (42). It is easily seen from the above discus- sion that, for any fixed VROA > 0 and for fixed bounds λα, λσ, c12,VROA > 0 can always be ensured if cpeε¯ is sufficiently small, i.e., if the bound c¯pe is sufficiently small.	□ Theorem 3 shows that the closed loop of the proposed data-

2

γλmin(P )

≤

driven MPC scheme admits a (practical) Lyapunov function, which converges robustly and exponentially to a set, whose size shrinks with the noise level. Since ξt  2  1   Vt due to (11), this implies practical exponential stability of the equilibrium ξ = 0. The result requires that the noise level ε¯ is small, the amount of persistence of excitation is large compared to the noise level (i.e., cpeε¯ is small), and the regularization pa- rameters are chosen suitably. Concerning the latter requirement, λα cannot be chosen arbitrarily large, which can be explained by noting that the optimal α is usually not zero, even in the noise-free case. On the other hand, λα cannot be too close to zero since solutions α(t) of (6a) are not unique and large choices of α(t) amplify the influence of the noise in y˜d on the prediction accuracy. Further, λσ has to be chosen sufficiently large to ensure stability, but not arbitrarily large for a fixed noise level. To be more precise, λαcpeε¯ and λσcpeε¯2 have to be small, i.e., for a fixed cpe, choosing the regularization parameters too large deteriorates the robustness of the scheme w.r.t. the noise level. One can show that the theoretical properties in Theorem 3

an upper bound on the noise level ε¯ is required. While these ingredients suffice to apply the proposed scheme,

≤

computing bounds as in (28) and (29) is a difficult task in practice. Theorem 3 should be interpreted as a qualitative result which illustrates 1) the influence of the regularization parameters on stability and robustness of the presented MPC scheme and 2) that large persistence of excitation (compared to the noise level) increases the region of attraction and reduces the tracking error. Further, many of the employed bounds rely on conservative esti- mates such as (a + b)2  2a2 + 2b2. In principle, it is possible to improve some of the quantitative estimates at the price of a more involved notation. Nevertheless, such improved estimates may lead to meaningful, nonconservative, verifiable conditions on the noise level ε¯ for closed-loop stability, and are, therefore, an interesting issue for future research.

Remark 7: In the nominal MPC scheme (3) as well as in its robust modification (6), the data (ud, yd) used for prediction is fixed. Alternatively, one may update the data using online measurements, given that the closed loop is persistently exciting. Indeed, we believe that one of the main advantages of the proposed scheme is its ability to cope (locally) with nonlin- ear components of the unknown system. Nonlinear dynamical systems are in general difficult to identify and thus, the proposed approach may be simpler than a model-based MPC scheme

with prior system identification. As illustrated in [21] with an application of a similar MPC scheme to a nonlinear stochastic quadcopter system, the approach is already applicable in practice to time-varying or nonlinear dynamics without updating the data online. Providing theoretical guarantees for the application of

0.017  0.001

⎡	⎤

+	u

⎢	⎥

0.001  0.023

⎦

⎣

0	0.061	k

0.072	0

the proposed scheme to a nonlinear system is an interesting and relevant problem for future research.

Similar to the nominal MPC scheme, it is easy to see that the

yk =

1  0  0  0

"	#

0  1  0  0

xk.

only free decision variables of Problem (6) are α(t) and σ(t) with at least m(L + 2n)+ n and p(L + n) free parameters, respec- tively (cf. Remark 1). On the contrary, to implement a model- based MPC scheme (with state measurements), mL parameters are required. When neglecting the constraint (6d) (cf. Remark 3),

For the following application of the robust data-driven MPC scheme, the system matrices are unknown and only measured input–output data is available. The control goal is tracking of the setpoint of the linearized system

the slack variable σ(t) can be eliminated from (6) by directly pe-

s	s	 "1#

"0.65#!

—

nalizing the norm of the model mismatch y¯(t)	HL+n(y˜d)α(t)

in the cost. Hence, considering the minimal amount of data

(u ,y ) =

,

1	0.77

required for persistence of excitation, Problem (6) has roughly the same number of decision variables as a model-based MPC problem. In contrast to the nominal case, however, Theorem 3 implies that larger data horizons N are beneficial for the the- oretical properties of the proposed scheme as they typically decrease the constant cpe. On the other hand, increasing values for N also lead to an increasing online complexity of (6) since

∈

α(t)	RN−L−n+1, i.e., the presented MPC approach allows

for a tradeoff between computational complexity and desired

closed-loop performance by appropriately selecting N .

On the contrary, the performance of identification-based MPC typically improves if larger amounts of data are employed, whereas the online complexity is independent of N . However, while the scheme presented in this article provides end-to-end guarantees for the closed loop using noisy data of finite length, the derivation of nonconservative estimation bounds on sys- tem parameters from such data, which would be required for guarantees in model-based MPC, is difficult in general and an active field of research [36], [37]. An extensive quantitative comparison of model-based MPC and the proposed data-driven MPC in theory and for practical examples is an interesting issue for future research.

V.  Example

In this section, we apply the robust data-driven MPC scheme of Section IV to a four tank system, which has been considered in [38]. This system is well-known as a real-world example, which is open-loop stable, but can be destabilized by an MPC without terminal constraints if the prediction horizon is too short. Similarly, we show in this section that our proposed scheme is able to track a specified setpoint, whereas a scheme without terminal constraints as suggested in [20]–[22] leads to an unstable closed loop, unless it is suitably modified.

We consider a linearized version of the system from [38], which takes the form

⎡0.921	0	0.041	0  ⎤

which is readily shown to satisfy the dynamics. We consider no constraints on the input or the output. In an open-loop experi- ment, an input–output trajectory of length N = 400 is measured, where the input is chosen randomly from the unit interval, i.e., ud [ 1, 1]2, and the output is subject to uniformly distributed additive measurement noise with bound ε¯ = 0.002. The online measurements used to update the initial conditions (6b) in the MPC scheme are subject to the same type of noise.

k

∈ −

We choose L = 30 for the prediction horizon as well as the following design parameters

Q = 3 · Ip, R = 10−4Im, λσ = 1000, λαε¯ = 0.1.

The closed-loop output resulting from the application of Prob- lem (6) in a one-step MPC scheme is displayed in Fig. 2. It can be seen that the control goal is fulfilled, with only slight deviations from the desired equilibrium. On the other hand, if the same scheme without terminal constraints is applied to the system, then the closed loop is unstable and diverges with the chosen parameters for both a one-step and an n-step MPC scheme (cf. again Fig. 2). This confirms our initial motivation that rigorous guarantees are indeed desirable for data-driven MPC methods, in particular when they are applied to practical systems. Furthermore, it can also be observed in Fig. 2 that an n-step version of the proposed MPC scheme with terminal equality constraints yields slightly better tracking accuracy, compared to the one-step scheme. We note that, with the above choice of parameters, the nonconvex constraint (6d) is automatically satisfied without enforcing it explicitly (cf. Remark 3).

≤	≤

Theorem 3 gives qualitative guidelines for the tuning of the design parameters to guarantee robust stability. In the following, we analyze the influence of various parameters on the closed- loop behavior. Theorem 3 requires that the regularization param- eters lie within specific bounds. This is confirmed for the present example, where the MPC scheme achieves desirable closed-loop performance similar to Fig. 2 as long as 0.05  λαε¯  0.5. If λα is chosen too low, then the closed loop is unstable since the

norm of α∗(t) and hence the amplification of the measurement

noise in (6a) is too large. On the contrary, if λα is chosen too

=

x

⎣

⎦

⎢	⎥

xk+1

0	0.918	0	0.033

0	0	0.924	0	k

0	0	0	0.937

large, then the asymptotic tracking error increases since the cost term λαε¯α∗(t)  2 dominates over the tracking cost. Similarly, if λσ < 500, then the closed loop may be unstable since we did not

or decrease, depending on the particular noise instance, and generally increases with the noise level ε¯. This confirms again the analysis of Section IV, which showed exponential stability of a set which grows with the noise level. Finally, if the norm of the data input ud increases (i.e., cpe decreases), then the tracking error decreases.

Fig. 2. Closed-loop output, resulting from the application of the robust data-driven MPC scheme with terminal equality constraints in a one-step fashion (TEC), in an n-step fashion (TEC, n-step), and without terminal equality constraints in a one-step fashion (UCON). (a) Closed-loop out- put y1. (b) Closed-loop output y2.

consider the constraint (6d) and therefore the slack variable is too large, which has a negative impact on the prediction accuracy. An upper bound on λσ beyond which the closed-loop behavior is undesirable could not be observed for the present example. Further, if the input weighting R is chosen too low, then the robustness with respect to the noise deteriorates, which can be explained via the bound (41), which grows with 1/λmin(R). If the input weighting is chosen large enough, then also an MPC scheme without terminal constraints stabilizes the desired equilibrium.

≤	≤

Regarding the knowledge of the system order n = 4, it suf- fices if an upper bound on n is available, i.e., if for instance n = 10 is used in (6). If the system order is assumed lower than n = 4, then the closed loop can be unstable. The prediction hori- zon L can be chosen (roughly) between 7 L 70. The upper bound can be explained by noting that a larger L implies that the constant cpe increases [compare the discussion after (9)] and therefore, the asymptotic tracking error increases. On the other hand, the lower bound is due to the terminal equality constraints, which require local controllability. Moreover, the steady-state tracking error, which can be seen e.g., in Fig. 2(b), may increase

VI.  Conclusion

In the present article, we proposed and analyzed a novel MPC scheme with terminal equality constraints, which uses only past measured data for the prediction, without any prior system identification step. We showed that, for a low noise amplitude, for a large ratio between persistence of excitation and the noise level, and for suitably tuned parameters, the closed loop in an n-step MPC scheme is recursively feasible and practically exponentially stable w.r.t. the noise level. To the best of our knowledge, we have provided the first analysis regarding recursive feasibility and stability for a purely data- driven (model-free) MPC scheme. Further, the analysis provides qualitative guidelines to choose the design parameters, and it illustrates the influence of other parameters, such as a persistence of excitation bound, on the region of attraction. While the MPC scheme is simple to implement, its analysis is challenging since we consider two sorts of noise: 1) additive output noise and 2) in the prediction model, similar to a multiplicative, parametric error in model-based MPC. In an application to a practical example, we showed that the proposed MPC scheme guarantees stability, whereas an existing data-driven MPC scheme without terminal constraints leads to an unstable closed loop.

Several topics for future research are left open. Extensions of

the presented data-driven MPC approach to online optimization over artificial equilibria and robust output constraint satisfaction are provided in the recent works [27] and [31], respectively. Another extension, which would be highly interesting but also challenging, is the development of data-driven MPC schemes for nonlinear systems with meaningful closed-loop guarantees. Finally, many of the bounds employed in our proofs are con- servative, and improving them may lead to less conservative, verifiable conditions on the admissible noise level for closed- loop stability.

References

- Z.-S. Hou and Z. Wang, “From model-based control to data-driven control: Survey, classification and perspective,” Inf. Sci., vol. 235, pp. 3–35, 2013,

- B. Recht, “A tour of reinforcement learning: The view from continuous control,” Annu. Rev. Control, Robot., Auton. Syst., 2018.

- J. B. Rawlings, D. Q. Mayne, and M. M. Diehl, Model Predictive Control: Theory, Computation, and Design, 2nd ed. Portland, OR, USA: Nob Hill Pub, 2017.

- L. Ljung, System Identification: Theory for the User. Englewood Cliffs, NJ, USA: Prentice-Hall, 1987.

- V. Adetola and M. Guay, “Robust adaptive MPC for constrained uncertain nonlinear systems,” Int. J. Adaptive Control Signal Process., vol. 25, no. 2,

pp. 155–167, 2011.

- A. Aswani, H. Gonzalez, S. S. Sastry, and C. Tomlin, “Provably safe and robust learning-based model predictive control,” Automatica, vol. 49, no. 5, pp. 1216–1226, 2013.

- M. Tanaskovic, L. Fagiano, R. Smith, and M. Morari, “Adaptive receding horizon control for constrained MIMO systems,” Automatica, vol. 50, no. 12, pp. 3019–3029, 2014.

- F. Berkenkamp, M. Turchetta, A. Schoellig, and A. Krause, “Safe model- based reinforcement learning with stability guarantees,” in Proc. Adv. Neural Inf. Process. Syst., 2017, pp. 908–918.

- M. Zanon and S. Gros, “Safe reinforcement learning using robust MPC,”

arXiv preprint arXiv:1906.04005.

- L. Hewing, J. Kabzan, and M. N. Zeilinger, “Cautious model predictive control using Gaussian process regression,” IEEE Trans. Control Syst. Technol., to be published, doi: 10.1109/TCST.2019.2949757.

- J. R. Salvador, D. M. de la Peña, T. Alamo, and A. Bemporad, “Data-based predictive control via direct weight optimization,” in Proc. IFAC Conf. Nonlinear Model Predictive Control, 2018, pp. 356–361.

- J. C. Willems, P. Rapisarda, I. Markovsky, and B. De Moor, “A note on persistency of excitation,” Syst. Control Lett., vol. 54, pp. 325–329, 2005.

- T. Katayama, Subspace Methods for System Identification. Berlin, Ger- many: Springer, 2005.

- I. Markovsky, J. C. Willems, P. Rapisarda, and B. L. M. D. Moor, “Algorithms for deterministic balanced subspace identification,” Automat- ica, vol. 41, no. 5, pp. 755–766, 2005.

- I. Markovsky and P. Rapisarda, “Data-driven simulation and control,” Int.

J. Control, vol. 81, no. 12, pp. 1946–1959, 2008.

- J. Berberich and F. Allgöwer, “A trajectory-based framework for data- driven system analysis and control,” in Proc. Eur. Control Conf., to be published, arXiv:1903.10723.

- C. De Persis and P. Tesi, “Formulas for data-driven control: Stabilization, optimality and robustness,” IEEE Trans. Autom. Control, vol. 65, no. 3,

pp. 909–924, Mar. 2020.

- A. Romer, J. Berberich, J. Köhler, and F. Allgöwer, “One-shot verification of dissipativity properties from input-output data,” IEEE Control Syst. Lett., vol. 3, no. 3, pp. 709–714, Jul. 2019.

- H. J. van Waarde, J. Eising, H. L. Trentelman, and M. Kanat Camlibel, “Data informativity: A new perspective on data-driven analysis and con- trol,” to be published, arXiv:1908.00468.

- H. Yang and S. Li, “A data-driven predictive controller design based on reduced Hankel matrix,” in Proc. 10th Asian Control Conf., 2015, pp. 1–7.

- J. Coulson, J. Lygeros, and F. Dörfler, “Data-enabled predictive control: in the shallows of the DeePC,” in Proc. 18th Eur. Control Conf., 2019,

pp. 307–312.

- J. Coulson, J. Lygeros, and F. Dörfler, “Regularized and distributionally robust data-enabled predictive control,” in Proc. 58th IEEE Conf. Decision Control, 2019, pp. 2696–2701.

- L. Grüne and J. Pannek, Nonlinear Model Predictive Control. Berlin, Germany: Springer, 2017.

- L. Grüne, “NMPC without terminal constraints,” in Proc. IFAC Conf. Nonlinear Model Predictive Control, 2012, pp. 1–13.

- D. Q. Mayne, J. B. Rawlings, C. V. Rao, and P. O. M. Scokaert, “Con- strained model predictive control: Stability and optimality,” Automatica, vol. 36, no. 6, pp. 789–814, 2000.

- D. Limón, I. Alvarado, T. Alamo, and E. F. Camacho, “MPC for tracking piecewise constant references for constrained linear systems,” Automatica, vol. 44, no. 9, pp. 2382–2387, 2008.

- J. Berberich, J. Köhler, M. A. Müller, and F. Allgöwer, “Data-driven tracking MPC for changing setpoints,” in Proc. IFAC World Congr., to be published, arXiv:1910.09443.

- A. Bemporad, M. Morari, V. Dua, and E. N. Pistikopoulos, “The explicit linear quadratic regulator for constrained systems,” Automatica, vol. 38, no. 1, pp. 3–20, 2002.

- C. Cai and A. R. Teel, “Input–output-to-state stability for discrete-time systems,” Automatica, vol. 44, no. 2, pp. 326–336, 2008.

- G. Grimm, M. J. Messina, S. E. Tuna, and A. R. Teel, “Model predictive control: For want of a local control Lyapunov function, all is not lost,” IEEE Trans. Autom. Control, vol. 50, no. 5, pp. 546–558, May 2005.

- J. Berberich, J. Köhler, M. A. Müller, and F. Allgöwer, “Robust constraint satisfaction in data-driven MPC,” to be published, arXiv:2003.06808.

- L. Grüne and V. G. Palma, “Robustness of performance and stability for multistep and updated multistep MPC schemes,” Discrete Continuous Dynamical Syst., vol. 35, no. 9, pp. 4385–4414, 2015.

- K. Worthmann, M. W. Mehrez, G. K. I. Mann, R. G. Gosine, and J. Pannek, “Interaction of open and closed loop control in MPC,” Automatica, vol. 82,

pp. 243–250, 2017.

- B. Kouvaritakis and M. Cannon, Model Predictive Control: Classical, Robust and Stochastic. Berlin, Germany: Springer, 2016.

- S. Yu, M. Reble, H. Chen, and F. Allgöwer, “Inherent robustness properties of quasi-infinite horizon nonlinear model predictive control,” Automatica, vol. 50, no. 9, pp. 2269–2280, 2014.

- N. Matni, A. Proutiere, A. Rantzer, and S. Tu, “From self-tuning regulators to reinforcement learning and back again,” arXiv preprint arXiv:1906.11392

- N. Matni and S. Tu, “A tutorial on concentration bounds for system identification,” arXiv preprint arXiv:1906.11395.

- T. Raff, S. Huber, Z. K. Nagy, and F. Allgöwer, “Nonlinear model pre- dictive control of a four tank system: An experimental stability study,” in Proc. IEEE Int. Conf. Control Appl., 2006, pp. 237–242.

Julian Berberich received the master’s degree in engineering cybernetics from the University of Stuttgart, Stuttgart, Germany, in 2018.

Since 2018, he has been the Ph.D. Student with the Institute for Systems Theory and Auto- matic Control under supervision of Prof. Frank Allgöwer and a member of the International Max-Planck Research School (IMPRS). His re- search interests are in the area of data-driven system analysis and control.

Johannes Köhler received the master’s degree in engineering cybernetics from the University of Stuttgart, Stuttgart, Germany, in 2017.

During his studies, he spent three months at Harvard University, Na Li’s Research Lab. He has since been a Doctoral Student with the Insti- tute for Systems Theory and Automatic Control under the supervision of Prof. Frank Allgöwer and a member of the Graduate School Soft Tissue Robotics at the University of Stuttgart. His research interests are in the area of model predictive control.

Matthias A. Müller the M.S. degree in electrical and computer engineering from the University of Illinois at Urbana-Champaign, Champaign, IL, USA, in 2009, the Diploma in engineering cybernetics in 2009, and the Ph.D. degree in mechanical engineering, from the University of Stuttgart, Germany, in 2014.

Since 2019, he is the Director with the Insti- tute of Automatic Control and a Full Professor with the Leibniz University Hannover, Germany.

His research interests include nonlinear control and estimation, model predictive control, and data-/learning-based con- trol, with application in different fields including biomedical engineering.

Dr. Müller received the 2015 European Ph.D. Award on control for complex and heterogeneous systems.

Frank Allgöwer studied engineering cybernet- ics and applied mathematics from Stuttgart and the University of California, Los Angeles (UCLA), respectively, and received the Ph.D. de- gree from the University of Stuttgart, Stuttgart, Germany.

Since 1999, he has been the Director of the Institute for Systems Theory and Automatic Control and a Professor with the University of Stuttgart. His research interests include net- worked control, cooperative control, predictive

control, and nonlinear control with application to a wide range of fields including systems biology.

Prof. Allgöwer serves as the President of the International Federation of Automatic Control (IFAC) from 2017–2020 and since 2012, he has been the Vice President of the German Research Foundation DFG.