Choose Wisely: Data-driven Predictive Control for Nonlinear Systems Using Online Data Selection

### Joshua Näf

ETH Zurich Zurich, Switzerland naefjo@ethz.ch

### Keith Moffat

ETH Zurich Zurich, Switzerland kmoffat@ethz.ch

### Jaap Eising

ETH Zurich Zurich, Switzerland jeising@ethz.ch

Florian Dörfler ETH Zurich Zurich, Switzerland dorfler@ethz.ch

# Abstract

This paper proposes Select-Data-driven Predictive Control (Select-DPC), a new method for controlling nonlinear systems using output-feedback for which data are available but an explicit model is not. At each timestep, Select-DPC employs only the most relevant data to implicitly linearize the dynamics in “trajectory space”. Then, taking user-defined output constraints into account, it makes control decisions using a convex optimization. This optimal control is applied in a receding-horizon manner. As the online data-selection is the core of Select-DPC, we propose and verify both norm-based and manifold-embedding-based selection methods. We evaluate Select-DPC on three benchmark nonlinear system simulators—rocket- landing, a robotic arm and cart-pole inverted pendulum swing-up—comparing them with standard Data-enabled Predictive Control (DeePC) and Time-Windowed DeePC methods, and find that Select-DPC outperforms both methods.

arXiv:2503.18845v2 [eess.SY] 22 May 2025

Introduction

Model-based methods have established their value in the Reinforcement Learning (RL) community, even for highly nonlinear systems Moerland et al. (2022). From a dif- ferent approach, the Control community has developed interest in “direct” Data-driven Predictive Control meth- ods Dörfler et al. (2023), which typically make Linear, Time-Invariant (LTI) assumptions. This paper proposes Select-Data-driven Predictive Control (Select-DPC), a di- rect Data-driven Predictive Control method for nonlinear systems.

Output or safety constraints are challenging for standard RL methods, which learn the constraints by exhaustive trial and error Sutton & Barto (1998). Output-constrained systems have traditionally been the domain of Model Pre- dictive Control (MPC) Morari & H. Lee (1999), which solves a receding-horizon optimal control problem based on an externally-provided model with an online optimiza- tion. In contrast, RL has favored planning methods that are unconstrained and “gradient-free” and, instead of requir- ing an externally-provided model, use state-action queries of the system. Recently, planning using forward passes through generative or world models have been used to re- duce RL sample complexity requirements Moerland et al.

Preprint. Under review.

Select-DeePC (ours) Full Data DeePC

Windowed DeePC Setpoint

Figure 1: Closed-loop trajectories of Select-DPC (ours), standard DeePC and Time-Windowed DeePC in the MuJoCo Reacher (left) and the Rocket Lander (right) Gym environments, as well as the corresponding open-loop predictions (dashed). In both environments, Select- DPC converges to the provided setpoint while standard and Time-Windowed DeePC fail to track the setpoints.

(2022). However, these zero-order-type planning methods usually have no structured way of incorpo- rating output constraints and require the user to offload this complexity into other parts of the method, such as the model itself or the cost.

The class of Data-driven Predictive Controllers (DPCs) construct feasible trajectories of an LTI system purely from data. Data-enabled Predictive Control (DeePC) Coulson et al. (2019), for instance, linearly combines input-output trajectories in the data set. DPC methods have demonstrated impressive results on a number of control problems. However, the LTI restriction is severe.

Select-DPC, proposed in this paper, extends DPC to the nonlinear setting by iteratively optimizing a convex quadratic program over an implicit linearization in “trajectory space” constructed from data. We define trajectory space as the high-dimensional space in which each dimension corresponds to an input or output over the course of a discrete-time, finite-length trajectory. Linearizing in trajectory space is achieved by selecting only trajectories that are close to the operating point in trajectory space. Thus, Select-DPC is a data-driven alternative to nonlinear MPC solved via Sequential Quadratic Programming (SQP) Diehl et al. (2002), which calculates a trajectory space linearization at each timestep from an externally provided model. SQP-MPC is a standard model-based optimal control tool, commonly used in both academia and industry Verschueren et al. (2020).

Explicit Constraint

Kernelized

Handling	DeePC

Model Free

Select-DPC

SQP-MPC

RL

Computationally Cheap (online)

Literature Review DeePC was first introduced by Coulson et al. (2019) as an alternative to the sequential, indirect system identification and MPC design pipeline. It uses Willems’ fundamental Lemma Willems et al. (2005) for deterministic LTI systems to generate trajectories of the system as opposed to an identified explicit linear model and is hence considered a direct method. Such predictions fit into the behavioral control framework, see, for instance, Willems (1991). For a survey of DeePC we refer the

interested reader to Markovsky et al. (2023).

Several extensions to DeePC have been proposed to handle nonlinear systems in a structured way as opposed to treating the observed nonlin- earity as measurement noise on a linear system. Berberich & Allgöwer (2024) provide a list of tailored formulations of the Fundamental Lemma to specific classes of nonlinear systems . These formulations enable effi- cient construction of a DeePC formulation for systems belonging to these specific system classes. However, this already imposes a strong prior on the global nonlinear structure of the dynamical process, which might not be available for an arbitrary system. Alternatively, Berberich et al. (2022) propose using a set of the most recent past observations along the current closed-loop trajectory to form the basis of a linear predictor, herein referred to as Time-Windowed DeePC. This predictor is continu-

Figure 2: Venn-diagram demonstrating	the strengths and weak- nesses of the discussed methods. Select-DPC combines the best of all three worlds by bridging the gap of explicit constraint handling without the requirement for Jacobians of the model achieved by direct methods.

ally updated as new data is obtained in a sliding window fashion, effectively forming a locally linear approximation of the underlying system. As this method only uses recent closed-loop data, it is practical only for systems with benign nonlinearities and with very high signal-to-noise ratios.

Other approaches to nonlinear-DeePC include reformulating the predictor in a kernelized fashion, enabling the use of nonlinear kernels Lian & Jones (2021); Huang et al. (2024), using nonlinear basis functions or reformulating the problem in a lifted state-space using Koopman operators Lazar (2023). While these methods enjoy greater flexibility due to their use of nonlinear function approximators, the resulting optimization becomes high-dimensional, nonlinear, and generally nonconvex, requiring increased online computational complexity and resulting in suboptimal outcomes.

The premise of DPC, solving an optimal control problem based purely on samples of a generative model without requiring gradient information, fits well into the landscape of planning algorithms in model-based RL. Optimization-based planning techniques suffer from accumulating prediction errors Moerland et al. (2022), exploding gradients due to backpropagation through time or model exploitation in regions of low data density Kurutach et al. (2018). Currently predominant planning techniques, herein called Sampling-Based MPC, are gradient-free methods that instead rely on sampling of the forward process model, making them computationally cheap to evaluate. Nagabandi et al. (2018) propose to generate rollouts of the forward dynamics from randomly sampled control inputs. Then, the rollouts are scored according to some reward function and the first action of the best trajectory is applied. In a similar vein, the cross-entropy method for optimization uses iterative refinement of the probability distribution over the control inputs to generate an optimal input sequence

Botev et al. (2013). A related algorithm is Model Predictive Path Integral Control (MPPI) Williams et al. (2017), which can be interpreted as an importance sampling scheme, weighing the control actions according to their optimality. Sampling-Based MPC in conjunction with RL methods have been successfully demonstrated in various algorithms and applications such as but not limited to Chua et al. (2018); Hafner et al. (2019); Nagabandi et al. (2019); Kahn et al. (2020). Other recent approaches include using a generative model to generate trajectories and then using an inverse model to predict the required action Ajay et al. (2023) or simply rely on random shooting and selecting the action sequence with highest reward Zhou et al. (2024). Figure 2 shows a schematic overview of the control strategies mentioned and their respective strengths and weaknesses.

Remark 1.1. While we study the performance of Select-DPC on a fixed data set, which is common in Data-driven Control literature, and hereby diverge from the common approach in Sampling-Based MPC where a simulator is queried at each decision moment, we note that Select-DPC can readily be extended to the simulator-based scenario by, e.g., using roll-outs from the simulator as the data set.

Contributions We propose Select-DPC, a data-driven quadratic program predictive controller for nonlinear systems which implicitly linearizes the system dynamics in the extended input-output space by selecting and using only the most relevant data at each decision moment from a fixed data set. It has the following benefits:

- In contrast with the nonlinear Data-driven Predictive Control methods in the literature such as basis function methods, Select-DPC produces trajectories which are consistent with the underlying nonlinear system using a convex quadratic programming solver, yielding a method that can be implemented online in fast dynamic environments.

- In contrast with model-based RL methods, Select-DPC allows users to explicitly program-in output constraints and change said constraints, as well as cost/reward function, a posteriori. This allows Select-DPC to be employed in changing safety conditions without requiring retraining or retuning.

- Select-DPC is easy to implement, requiring only a few lines of code for the end-user. Moreover it is conceptually easy to reason about and adapt due to its modular structure. In particular, it enables a rich choice of data-selection methods.

The core idea behind Select-DPC is the selection of a subset of data from a large data set at each decision moment. We introduce two methods for the data-selection problem, norm-based and manifold-embedding-based selection, and discuss their relevant strengths and weaknesses. Further- more, Select-DPC can be used with any DPC method such as Coulson et al. (2019); Breschi et al. (2023); Moffat et al. (2024); Favoreel & De Moor (1999).

Finally, we demonstrate the benefits of Select-DPC on three benchmark nonlinear systems including a planar rocket-landing, a two degree of freedom robotic arm, and a cart-pole inverted pendulum, comparing it to standard DeePC and a Time-Windowed DeePC approach and find that Select-DPC outperforms both in terms of closed-loop cost and prediction accuracy.

# Select-DPC

Select-DPC is a receding-horizon optimal control approach which determines inputs at each timestep using a predictive optimization. While it does not use a model in the classical sense, in the RL parlance, it could be interpreted as a “model-based” method Sutton & Barto (1998), while in the Data-driven Control community it is a “direct” method Dörfler et al. (2023).

Ð

Ð	{	}

Select-DPC builds upon linear DPC which solves a receding-horizon predictive control problem subject to input and output constraints. The novelty of DPC is that it achieves predictive control purely based on implicit predictions of input-output data without ever constructing an explicit predictor (a model) of future measurements given a set of inputs. DeePC, for instance, achieves this by linearly combining input-output trajectories τ of length L from a data set = τ0, . . . , τnd . The L-length trajectories τi are partitioned into a past trajectory τp,i and a future trajectory τf,i with lengths Tp and Tf respectively with Tp + Tf = L. The implicit predictor in DeePC works by pattern matching the past Tp input-output measurements of the current closed-loop trajectory to a linear combination of the τp,i in . Since this can be achieved by (potentially) many different linear combinations, the objective of DeePC is to then find a linear combination of future trajectories τf that minimizes a cost function subject to the pattern matching constraint. Several regularizing terms have been introduced to further bias the choice of said linear combinations. For more details, we refer to Appendix A.

Solving DeePC online using a large data set of trajectories is intractable as the solve time of the optimization problem scales superlinearly with the cardinality of the data set. Fur- thermore, in the presence of nonlinearities, the LTI assumption of DeePC is violated, re- sulting in suboptimal performance.  We now introduce Select-DPC, an algorithm that al- lows us to solve a nonlinear data-driven receding-horizon optimal control problem using a convex quadratic programming routine by choosing, at each solver iteration, a subset

of trajectories Ð˜ with cardinality card(Ð˜)  =  Ncols from Ð where Ncols  ≪ card (Ð).

Select-DPC is summarized in Algorithm 1. In between two real-time sampling instances, an iterative procedure is performed where, at every

Algorithm 1 Select-DPC

1: function Select-DPC(up, yp)

2:	while convergence criterion not met do

iteration, we select from a data set Ð a subset of	3:

τ˜  →  DPC.getLastPrediction()

Ð

trajectories ˜ that are “relevant” to the current

linearization point given by the open loop solu-	4:

D˜ → select(Ð, τ˜, N

cols)

˜

→

tion τ˜. While we loosely use the term relevance here, this mechanism will be discussed in more detail in Section 3. Select-DPC then solves a DPC problem using the subset of data. This is

5:	u	DPC.computeAction(up, yp, D)

6:	end while

7:	Return u0

8: end function

iterated until some convergence criterion is met, e.g. the open-loop prediction τ˜ converges. Notice that there is no restriction on the choice of DPC solver.

We now discuss a number of attributes of this proposed method.

Cost-agnostic Since the training process/data set generation is structurally independent of the control formulation, a posteriori cost or constraint modification is possible, allowing the method to be applied zero-shot to new problems without the need for retraining.

Hyperarameters The performance of Select-DPC depends on the tuning of several hyperparame- ters. First, the DPC specific hyperparameters, such as regularization terms and horizon lengths Tp, Tf. These can either be hand-tuned according to established heuristics Elokda et al. (2021) or optimized through gradient-descent methods Cummins et al. (2024) or black-box parameter tuning Berkenkamp et al. (2016). For DPC tuning guidelines, we refer to Markovsky et al. (2023). Select-DPC adds onto these the design parameters Ncols as well as the method of data selection. In the following section, we will investigate two different data-selection methods which subsample a specified number of trajectories from a large offline library of trajectories observed during “training.”

SQP-MPC The iterative refinement of the solution until convergence is inspired by the model-based SQP-MPC algorithm for nonlinear systems Diehl et al. (2002). We direct the interested reader to Appendix B for a detailed discussion of SQP-MPC and the relationship to Select-DPC. We note that Select-DPC’s implicit linearization is performed in trajectory space. In the model-based case, linearizing in trajectory space corresponds to a linear time-varying parametrization of the nonlinear dynamics for the MPC. This linear, time-varying parametrization contrasts with an LTI formulation in which the Jacobians are only evaluated at the current “state” y(k) and kept fixed along the horizon.

&

&

Computational complexity Solving the convex quadratic optimization problem at the basis of Select-DPC involves repeatedly solving a symmetric system of linear equations Stellato et al. (2020). Hence, we expect the algorithm to scale with (n3) where n indicates the number of optimization variables which is directly proportional to the number of trajectories Ncols selected from the full data set. Hence, from a computational complexity standpoint, we wish to perform the optimization with as few trajectories as possible. Additionally, the dominant computation in the data-selection is the sorting of the data according to the relevance to τ˜ which scales with (nd log nd), which we postulate is less significant than the computational complexity of the optimization problem. While Select-DPC solves a convex quadratic program and is smaller than full-data DPC or nonlinear DPC, it still requires more compute than Sampling-Based MPC methods. There exist DPC variants which reduce the number of optimization variables in the predictive control problem Favoreel & De Moor (1999); Breschi et al. (2023) but the comparison of these methods is outside the scope of this study. For an overview of existing methods we refer to Verheijen et al. (2023).

# Data Selection

At the heart of Select-DPC lies the data selection method, which selects data points (representing trajectories τi) from a large (offline) data set. Since the subproblem solved by Select-DPC relies on

Ð

linearly combining trajectories, the subset of data should exhibit predominantly linear dynamics. The suggested approach in the literature is to use a sliding window over past input-output measurements. We propose that ˜ should be chosen according to some spatial distance metric (in trajectory space) as opposed to temporal proximity (such as a sliding window) to the current linearization point τ˜.

In the following, we will present two selection methods, norm-based and manifold-embedding-based selection, while placing emphasis on the modular nature of Select-DPC. While norm-based selection is efficient to implement and allows the user to easily update the data set online, manifold-embedding- based selection enables the distance computation in a lower dimensional embedding space which lessens the impact of the curse of dimensionality but comes with increased offline computation cost.

Ð

Norm-based selection  Norm-based data selection selects from a data set  the Ncols clos- est (in space) trajectories by computing the norm of the differences between the open-loop so- lution τ˜ and the trajectories in the data set Ð.  An implementation is given in Algorithm 2.

While this approach is intuitive, it is common knowledge that norm-based distances fall flat in high dimensional spaces due to the curse of dimen-

Algorithm 2 Norm-based Data Selection

Ð

1: function NormDataSelection(Ð, τ˜, Ncols)

sionality Aggarwal et al. (2001). This method is no

2:	Ð˜ = sort({τi — τ˜  | i = 1, . . . , nd})

exception since we are computing norm-based dis- tances of trajectories, i.e. vectors with dimension (Tp + Tf)(m + p).

3:	Pick first [1, Ncols] from ˜

4: end function

Ð

·

Low Dimensional Representation using Manifold Learning The field of Manifold Learning (Nonlinear Dimensionality Reduction) concerns itself with the compression of high-dimensional data onto manifolds with lower intrinsic dimension than the original data by using (nonlinear) projection techniques Lee & Verleysen (2007); Jia et al. (2022). Manifold Learning techniques rely on the manifold hypothesis Fefferman et al. (2016), which postulates that the intrinsic dimensionality of a data set  is much lower than the dimensionality of the individual data points. For instance, if the system under consideration is a controllable and deterministic LTI system, then Willems’ Fundamental Lemma states that the trajectories span a manifold (in particular a subspace) that can at most have n + Tf m degrees of freedom Willems et al. (2005), which is generally less than the trajectory dimensionality (Tp + Tf)(m + p). Here, n denotes the dimension of the process’s latent state, p is the dimension of the measurement vector and m indicates the dimension of the input signal. Motivated by this observation, we wish to find a lower dimensional representation of the data where the similarities can be computed without incurring the curse of dimensionality.

Isomap, introduced by Tenenbaum et al. (2000), is an unsupervised manifold learning method which aims to find an embedding of the data in a Euclidean space which preserves geodesic distances of a neighborhood graph of the data constructed in the data space. This comes with increased computational cost compared to norm based data selection but can offer inter data point distances which more accurately reflect the nonlinear nature of the data manifold in trajectory space. Also notice that the bulk of this added cost is a one-time upfront cost (computing the embedding), which can be performed offline. For algorithmic details, see Appendix C.

·

Although this does not fully resolve the problem caused by the curse of dimensionality as the embedding dimension, i.e. the dimension where we need to compute distances, will still scale with at least with n + Tf m, it does offer a structured way of compressing the data to a reasonable dimension. Experiments showed that embedding dimensions lower than the minimum dimension required to represent a linear system already lead to diminishing returns in reconstruction error.

The pseudocode for manifold-embedding-based data selection using the Isomap algorithm is out- lined in Algorithm 3, the details of embed using Isomap can be found in Appendix C. Notice that

Algorithm 3 Manifold-Embedding Data Selection

1: function ManifoldDataSelection(Ð, τ˜, Ncols)

2:	Ðe → embed (Ð)

the embedding of the data set in line 2 can be	3:

cached/computed offline. Online, the algorithm	4:

τ˜e → embed (τ˜)

Ð

Ð˜e = sort({τe,i — τ˜e | i = 1, . . . , nd})

then proceeds to embed the query trajectory τ˜i and computes its similarities in the lower di- mensional embedding space. Then, similarly to

5:	Pick first [1, Ncols] from ˜e

6: end function

Algorithm 2, the relevant trajectories are returned in the trajectory space, after which the implicit predictor for DPC is constructed using the retrieved non-embedded data points.

# Results and Discussion

Simulation experiments were performed to validate the efficacy of Select-DPC and to benchmark it against classical DeePC Coulson et al. (2019), which utilizes the entire data set, as well as the Time-Windowed DeePC variant proposed by Berberich et al. (2022), which constructs the implicit predictor by continuously updating the data set with new input-output data and discarding the most out-of-date measurement. All experiments were performed on an Apple MacBook Pro with M2 Pro chip and 16GB of RAM. To speed up computation times, an LQ decomposition of the subset of data was performed in the experiments regarding closed-loop cost for large subset sizes Breschi et al. (2023). We use DeePC as the linear DPC solver in Select-DPC.

### Landing a Reusable Rocket

We use a planar vertical takeoff, vertical landing rocket to demonstrate the performance of Select-DPC on an open-loop unstable nonlinear system. We show that Select-DPC successfully copes with the nonlinear dynamics and manages to stabilize the system at a given setpoint. A custom environment for the gymnasium suite Kwiatkowski et al. (2024) was used as a simulation platform. For a detailed description of the environment, we refer to Cummins et al. (2024).

The right column in Figure 1 shows the simulator setup and specifically, in the bottom figure, three closed-loop rocket trajectories. One for Select-DPC, one for standard DeePC and one for Time- Windowed DeePC respectively. While both the standard DeePC and the Time-Windowed DeePC controllers diverge, Select-DPC manages to regulate the system to the desired setpoint and stabilize it there.

Ablation Study We compare the performance of Select-DPC using the two presented selection methods on two different data sets. Both data sets were collected using input sequences of the form

ut = a · ut−1 + b · n, n ∼ C(—1, 1).	(1)

≈

The data set IID Data uses (a, b) = (0, 1) and Random Walk Data uses (a, b) = (1, 0.1). To ensure that a large region of the trajectory space was covered by the collected data, 100 simulations were run with a maximum of 100 timesteps each, resulting in data sets with  5000 data points. As shown later, the level of nonlinearity in the data has a significant effect on the performance of standard DeePC. Due to the high inertia of the rocket, the IID Data predominantly exhibits behaviors that can be represented reasonably well by a linear model, and hence, standard DeePC can be applied. However, this comes at a performance penality as nonlinear effects are not accounted for. We observe that Select-DPC still outperforms standard DeePC on IID Data and that data selection can still lead to performance increases if the data set does not fully capture the nonlinear behavior of the underlying system. The random walk structure of the input sequence in Random Walk Data, on the other hand, ensures that the nonlinearities of the system are sufficiently excited. This makes standard DeePC fail but provides Select-DPC with more global information about the nonlinear system.

The comparisons of Select-DPC on the two data sets are two-fold. First, the resulting predictive accuracy of the selection methods are compared.  Furthermore, a comparison of the realized closed-loop cost is given as a function of selection method and data set. As a benchmark, random sketching of the full data set at each decision moment is used.

The predictive accuracy of the methods was com-	Predictive Accuracy on Rocket Data Sets

pared by computing the least squares solution of the predictor used in the DPC subproblem as a function of the number Ncols of trajectories in the selected set of data. For each trajectory in the holdout data set, constructed from trajectory- segments of a closed-loop trajectory, the Ncols

102

yf − yˆf

2

101

Random Walk Data

102	103

IID Data

102	103

closest trajectories (in spatial L1-norm or using Isomap-embedding) under the given selection method were selected and the prediction of fu- ture measurements yˆf, given the corresponding future input sequence uf was computed using Equation (11). The plots in Figure 3 show the cu- mulative residuals between the predicted future

Ncols

Selection Method

Random	L1	Isomap

Figure 3: Residual between the ground-truth and the least-squares prediction as a function of trajec- tories used in the predictor.

trajectory and the ground truth realization

yˆf — yf  2 over the entire validation set.

1.25

1.00

0.75

Closed-Loop Cost for the Rocket

IID Data

Solve Time [s]

100

Computation Times of Select-DPC

Random Walk Data

3

2

1

103

Ncols

10−1

200	400	600	800	1000

Ncols

Selection Method

Random	L1

Isomap

Selection	QP Solve	O(1)	O(n2)

Figure 4: (left) Closed-loop cost as a function of data subset cardinality on two different data sets. Instances where the selection methods realized infinite cost (due to instability) have been omitted and the remaining finite cost data points are shown. (right) Solve times of Select-DPC as a function of number of Hankel columns decomposed into data selection time and QP solve time.

Ð

In the extreme case where Ncols = card( ), each of the methods coincides with the least squares solution of the predictor in standard DeePC. We observe that both our methods of data selection (based on spatial L1-norm or Isomap-embedding) lead to lower cumulative residual on the holdout validation set when compared to using the full data set or random sketching. In particular, there is a sweet spot in the number Ncols of trajectories to use in the predictor.

The left plot of Figure 4 shows the closed-loop performance of Select-DPC on the setpoint tracking task depicted in the right column of Figure 1 on both IID Data and Random Walk Data. We again observe on both data sets that selecting data outperforms using the full data set in terms of closed-loop performance, as a lower cost could be achieved. Using IID Data, the reduction in closed-loop cost is 1.9 while using Random Walk Data, standard DeePC using the full data set failed to stabilize the rocket.

Computational Cost The right plot of Figure 4 shows the computation times of Select-DPC as a function of number of subselected trajectories. As postulated in Section 2, the dominant factor in the solve times is solving the quadratic program that scales at least quadratically with Ncols, while data selection is a constant time operation with respect to Ncols.

### Planar Robotic Manipulator

∈

∈

We use the Reacher environment Kwiatkowski et al. (2024), which simulates a planar robotic manipulator, to demonstrate the ability of Select-DPC to control a highly nonlinear system and successfully track reference setpoints. Furthermore, we show that Select-DPC is effective in coping with and respecting a posteriori specified output constraints. The kinematic chain of the manipulator consists of two rigid bodies that are connected by revolute joints which are each actuated by a torque, giving it an action space of u  R2. The measurement vector consists of the sine and cosine of both joint angles respectively, and the end-effector position in R2. Furthermore, we are also able to measure the angular velocity of each link, resulting in an observation space y  R8. This system is more challenging to control using DeePC (or any linear control method) since the end-effector position is dictated by the composition of two rotations. In this environment, it is not unreasonable to assume that the angles must exceed 90◦ in order to reach a target. Furthermore, for a given

end-effector position, the corresponding joint configuration is not necessarily unique.

Ablation Study  We again investigate the effect of number of selected trajectories Ncols, as well as selection strategy on the resulting closed loop cost. For the Reacher, data were collected using IID inputs. In total, 200 simulations were run with 200 steps each. Here, only IID inputs were collected instead of random walk because the system has lower inertia compared to the rocket and the nonlinearities could be excited more easily.

Select-DPC Closed-Loop Trajectories

Unconstrained Constrained

Cost

100

Closed-Loop Cost for the Reacher

y

10−1

102	103	104

Ncols

Selection Method

Random	L1	Isomap

x

Figure 5: (left) Closed-loop trajectories for a constrained (dark blue) and unconstrained (blue) DPC subproblem. (right) Comparing the closed-loop cost incurred as a function of subset cardinality. Clearly there is a sweet spot in how many trajectories should be selected for the predictions.

Select-DPC was successfully able to track an end-effector reference as shown in the left column of Figure 1. While Select-DPC is able to converge to the desired setpoint in 1 s, standard DeePC fails to come close to the reference during the entire simulation horizon. Clearly, the open-loop predictions do not align with the closed-loop behavior, indicating that using the full data set results in bad predictive accuracy. Time-Windowed DeePC also fails to converge to the setpoint in the allowed simulation horizon.

The right plot of Figure 5 again shows the incurred closed-loop cost as a function of number of trajectories used in the implicit predictor. Similarly to the rocket, we observe a decrease in predictive accuracy and an increase in cost as the number of data points used in the predictor increases. While random sketching of the data set showed decent results in the rocket simulation, it fails completely in the reacher simulation as no accurate predictions could be generated, resulting in the controller not making any progress towards the setpoint and hence accumulating high closed-loop cost.

Zero-Shot Generalizability of Select-DPC The left plot of Figure 5 shows two closed-loop trajectories of Select-DPC where the controller was tasked with tracking a set of consecutive end- effector references. We observe that Select-DPC is able to generalize to different tasks across the entire trajectory manifold of the Reacher by reusing the same data set and without structural changes to the controller. While one simulation was left unconstrained, a constraint of y < 0.1 was added to the DPC subproblem of the other one, indicated by the red region. The structure of Select-DPC effectively decouples the dynamics learning/training process from the controller design. Indeed, both the constrained and unconstrained tasks use the same data set which is unaware of the structure of the predictive control problem. This allows us to easily adapt the cost function or constraint terms for the task at hand in zero-shot fashion without requiring task specific data or additional online learning.

### Cart-Pole inverted pendulum swing-up

∈

∈

Finally, we show that Select-DPC is capable of performing a cart-pole inverted pendulum swing-up, which is a standard benchmark for nonlinear MPC methods. This test demonstrates that Select-DPC successfully plans trajectories for the nonlinear dynamics using its implicit linearization in trajectory- space. The input u R of the inverted pendulum is a force acting on the cart along the x-axis. Furthermore, the measurement y  R5 consists of the cart position, the sine and cosine of the angle between the pendulum and the vertical axis, as well as linear and angular velocities of the cart and pendulum respectively.

Due to the unstable nature of the setpoint and the system starting in a stable equilibrium position, data was gathered in closed-loop. Specifically, the data set consists of 200 demonstrations of successful but suboptimal swing-ups, collected using an energy based feedback controller Chatterjee et al. (2002).

The left plot of Figure 6 shows the closed-loop trajectories of both Select-DPC and standard DeePC when tasked with tracking a setpoint at the upward facing, unstable equilibrium while starting from the downward facing stable equilibrium. While standard DeePC is unable to perform the

Closed Loop Trajectory

Number of Linearly Combined Trajectories

y

# Nonzero Elements

20

10

0	50	100	150

x	Simulation Timestep

Select-DPC	Standard DeePC

Figure 6: (left) Closed-loop trajectory for an inverted pendulum swing-up. Select-DPC (blue) is successful in transitioning from the downward facing stable equilibrium to the upward facing unstable equilibrium. Standard DeePC (orange) on the other hand fails to transition between the equilibria. The dashed blue line indicates the open-loop prediction of Select-DPC at the four snapshot time instances, t=15, 22, 32 and 120. (right) Number of trajectories used to construct the final prediction at each sampling instant of the inverted pendulum swing-up.

swing-up and oscillates around the stable equilibrium, Select-DPC is successfully able to learn from the demonstrations in the data set and performs a swing-up to the desired vertical position and subsequently stabilizes the pendulum there. It is worthwhile to point out that the controller does not simply select the top trajectory in the selected data set. Instead, Select-DPC linearly combines several trajectory segments at each iteration of the solver to produce the swing-up (c.f. right plot of Figure 6). This is also evident from the fact that the horizon in the predictive controller is shorter than the episode length, thus the solver never has access to a full swing-up trajectory.

# Conclusion

This paper introduces Select-DPC, a novel approach to constraint-aware Data-driven Predictive Control for nonlinear systems. At each time instance, the method pre-processes the data set to determine the most relevant data, then passes the most relevant data to a convex optimization which determines the optimal control for the future horizon. The optimal control for the given timestep is implemented, and the algorithm progresses in a receding horizon fashion.

In contrast to other DPC methods, Select-DPC can be implemented easily, scales favorably with the number of collected data-points and increases performance outside of the linear domain. Furthermore, compared to Sampling-Based MPC, Select-DPC can explicitly handle output constraints.

We validated this in a variety of forms and on a set of three simulation environments. In the rocket simulator environment, we showed that for one of the data sets considered, Select-DPC cut closed-loop cost by a factor of 2. For the other data set, our method succeeded, where previous methods crashed. In the second set of simulations in the Reacher environment, we enabled zero-shot constrained setpoint tracking of a reference signal in a robotic reacher simulation. Again, this was not achievable using previous DPC formulations. Finally, in the last set of simulations of a cart-pole inverted pendulum, we demonstrated successful transitioning between two different equilibria of the system by performing a pendulum swing-up.

A direction of future work is to investigate different selection methods, such as structure-exploiting selection methods or selection methods based on an information criterion. Another direction is to replace Select-DPC’s large training data set with a data-driven forward simulator of the dynamics (i.e., a “world model”), resulting in a constraint-aware alternative to Sampling-Based MPC.

# References

Aggarwal, C. C., Hinneburg, A., and Keim, D. A. On the Surprising Behavior of Distance Metrics in High Dimensional Space. In Goos, G., Hartmanis, J., Van Leeuwen, J., Van Den Bussche, J.,

and Vianu, V. (eds.), Database Theory — ICDT 2001, volume 1973, pp. 420–434. Springer Berlin Heidelberg, Berlin, Heidelberg, 2001. Series Title: Lecture Notes in Computer Science.

Ajay, A., Du, Y., Gupta, A., Tenenbaum, J., Jaakkola, T., and Agrawal, P. Is Conditional Generative Modeling all you need for Decision-Making?, July 2023. arXiv:2211.15657 [cs].

Berberich, J. and Allgöwer, F. An Overview of Systems-Theoretic Guarantees in Data-Driven Model Predictive Control. October 2024. Publisher: Annual Reviews.

Berberich, J., Köhler, J., Müller, M. A., and Allgöwer, F. Linear tracking MPC for nonlinear systems Part II: The data-driven case. IEEE Transactions on Automatic Control, 67(9):4406–4421, 2022.

Berkenkamp, F., Schoellig, A. P., and Krause, A. Safe Controller Optimization for Quadrotors with Gaussian Processes. In 2016 IEEE International Conference on Robotics and Automation (ICRA),

pp. 491–496, 2016.

Botev, Z. I., Kroese, D. P., Rubinstein, R. Y., and L’Ecuyer, P. Chapter 3 - The Cross-Entropy Method for Optimization. In Rao, C. R. and Govindaraju, V. (eds.), Handbook of Statistics, volume 31 of Handbook of Statistics, pp. 35–59. Elsevier, January 2013.

Breschi, V., Chiuso, A., and Formentin, S. Data-driven predictive control in a stochastic setting: a unified framework. Automatica, 152:110961, June 2023.

Chatterjee, D., Patra, A., and Joglekar, H. K. Swing-up and stabilization of a cart–pendulum system under restricted cart track length. Systems & Control Letters, 47(4):355–364, 2002.

Chua, K., Calandra, R., McAllister, R., and Levine, S. Deep Reinforcement Learning in a Handful of Trials using Probabilistic Dynamics Models, November 2018. arXiv:1805.12114 [cs].

Coulson, J., Lygeros, J., and Dörfler, F. Data-Enabled Predictive Control: In the Shallows of the DeePC. In 2019 18th European Control Conference (ECC), pp. 307–312, 2019.

Cummins, M., Padoan, A., Moffat, K., Dorfler, F., and Lygeros, J. DeePC-Hunt: Data-enabled Predictive Control Hyperparameter Tuning via Differentiable Optimization, December 2024. arXiv:2412.06481 [math].

Diehl, M., Bock, H. G., Schlöder, J. P., Findeisen, R., Nagy, Z., and Allgöwer, F. Real-time optimization and nonlinear model predictive control of processes governed by differential-algebraic equations. Journal of Process Control, 12(4):577–585, 2002.

Dörfler, F., Coulson, J., and Markovsky, I. Bridging Direct and Indirect Data-Driven Control Formulations via Regularizations and Relaxations. IEEE Transactions on Automatic Control, 68 (2):883–897, February 2023. Conference Name: IEEE Transactions on Automatic Control.

Elokda, E., Coulson, J., Beuchat, P. N., Lygeros, J., and Dörfler, F. Data-enabled predictive control for quadcopters. International Journal of Robust and Nonlinear Control, 31(18):8916–8936, 2021.

Favoreel, W. and De Moor, B. SPC: Subspace Predictive Control. IFAC Proceedings Volumes, 32, January 1999.

Fefferman, C., Mitter, S., and Narayanan, H. Testing the manifold hypothesis. Journal of the American Mathematical Society, 29(4):983–1049, October 2016.

Hafner, D., Lillicrap, T., Fischer, I., Villegas, R., Ha, D., Lee, H., and Davidson, J. Learning Latent Dynamics for Planning from Pixels, June 2019. arXiv:1811.04551 [cs].

Huang, L., Lygeros, J., and Dörfler, F. Robust and Kernelized Data-Enabled Predictive Control for Nonlinear Systems. IEEE Transactions on Control Systems Technology, 32(2):611–624, March 2024. Conference Name: IEEE Transactions on Control Systems Technology.

Jia, W., Sun, M., Lian, J., and Hou, S. Feature dimensionality reduction: a review. Complex & Intelligent Systems, 8(3):2663–2693, 2022.

Kahn, G., Abbeel, P., and Levine, S. BADGR: An Autonomous Self-Supervised Learning-Based Navigation System, April 2020. arXiv:2002.05700 [cs].

Kurutach, T., Clavera, I., Duan, Y., Tamar, A., and Abbeel, P. Model-Ensemble Trust-Region Policy Optimization, October 2018. arXiv:1802.10592 [cs].

Kwiatkowski, A., Towers, M., Terry, J., Balis, J. U., Cola, G. D., Deleu, T., Goulão, M., Kallinteris, A., Krimmel, M., KG, A., Perez-Vicente, R., Pierré, A., Schulhoff, S., Tai, J. J., Tan, H., and Younis, O. G. Gymnasium: A Standard Interface for Reinforcement Learning Environments, October 2024. arXiv:2407.17032.

Lazar, M. Basis functions nonlinear data-enabled predictive control: Consistent and computationally efficient formulations, November 2023. arXiv:2311.05360 [cs, eess, math].

Lee, J. A. and Verleysen, M. (eds.). Nonlinear Dimensionality Reduction. Information Science and Statistics. Springer, New York, NY, 2007. ISBN 978-0-387-39350-6.

Lian, Y. and Jones, C. N. Nonlinear data-enabled prediction and control. In Proceedings of the 3rd Conference on Learning for Dynamics and Control, volume 144 of Proceedings of Machine Learning Research, pp. 523–534. PMLR, 07 – 08 June 2021.

Markovsky, I., Huang, L., and Dörfler, F. Data-Driven Control Based on the Behavioral Approach: From Theory to Applications in Power Systems. IEEE Control Systems, 43(5):28–68, October 2023.

Martinelli, A., Gargiani, M., Draskovic, M., and Lygeros, J. Data-Driven Optimal Control of Affine Systems: A Linear Programming Perspective. IEEE Control Systems Letters, 6:3092–3097, 2022. Conference Name: IEEE Control Systems Letters.

Moerland, T. M., Broekens, J., Plaat, A., and Jonker, C. M. Model-based Reinforcement Learning: A Survey, March 2022. arXiv:2006.16712 [cs].

Moffat, K., Dörfler, F., and Chiuso, A. The transient predictor. In 2024 IEEE 63rd Conference on Decision and Control (CDC), pp. 1871–1876, 2024.

Morari, M. and H. Lee, J. Model predictive control: past, present and future. Computers & Chemical Engineering, 23(4):667–682, May 1999.

Nagabandi, A., Kahn, G., Fearing, R. S., and Levine, S. Neural Network Dynamics for Model- Based Deep Reinforcement Learning with Model-Free Fine-Tuning. In 2018 IEEE International Conference on Robotics and Automation (ICRA), pp. 7559–7566, May 2018.

Nagabandi, A., Konoglie, K., Levine, S., and Kumar, V. Deep Dynamics Models for Learning Dexterous Manipulation, September 2019. arXiv:1909.11652 [cs].

Stellato, B., Banjac, G., Goulart, P., Bemporad, A., and Boyd, S. OSQP: an operator splitting solver for quadratic programs. Mathematical Programming Computation, 12(4):637–672, 2020.

Sutton, R. S. and Barto, A. G. Reinforcement Learning: An Introduction. MIT Press, Cambridge, MA, 1998.

Tenenbaum, J. B., Silva, V. D., and Langford, J. C. A Global Geometric Framework for Nonlinear Dimensionality Reduction. Science, 290(5500):2319–2323, December 2000.

Verheijen, P. C. N., Breschi, V., and Lazar, M. Handbook of linear data-driven predictive control: Theory, implementation and design. Annual Reviews in Control, 56:100914, January 2023.

Verschueren, R., Frison, G., Kouzoupis, D., Frey, J., van Duijkeren, N., Zanelli, A., Novoselnik, B., Albin, T., Quirynen, R., and Diehl, M. acados: a modular open-source framework for fast embedded optimal control, November 2020. arXiv:1910.13753 [math].

Willems, J. Paradigms and puzzles in the theory of dynamical systems. IEEE Transactions on Automatic Control, 36(3):259–294, 1991.

Willems, J. C., Rapisarda, P., Markovsky, I., and De Moor, B. L. M. A note on persistency of excitation. Systems & Control Letters, 54(4):325–329, April 2005.

Williams, G., Aldrich, A., and Theodorou, E. A. Model Predictive Path Integral Control: From Theory to Parallel Computation. Journal of Guidance, Control, and Dynamics, 40(2):344–357, February 2017.

Zhou, G., Swaminathan, S., Raju, R. V., Guntupalli, J. S., Lehrach, W., Ortiz, J., Dedieu, A., Lázaro- Gredilla, M., and Murphy, K. Diffusion Model Predictive Control, October 2024. arXiv:2410.05364 [cs].

# DeePC

Data-enabled Predictive Control (DeePC) solves a receding horizon optimal control problem based purely on data for a linear time-invariant system (LTI) of the form

yp(k — 1)

yf(k) = Fp up(k — 1) + Ffuf(k),	(2)

where u and y are partitioned into “past” and “future” quantities up, uf and yp, yf respectively. This means that the sequence of predicted future measurements, the past measurements used by the predictor, the sequence of past inputs applied to the system and the future control input sequence respectively given by

yf(k) :=  y(k)⊤	. . .  y(k + Tf — 1)⊤ ⊤ ∈ RTf·p,

uf(k) :=  u(k)⊤	. . .  u(k + Tf — 1)⊤ ⊤ ∈ RTf·m,

yp(k — 1) := y(k — Tp)⊤	. . .  y(k — 1)⊤ ⊤ ∈ RTp·p, and

up(k — 1) := u(k — Tp)⊤	. . .  u(k — 1)⊤ ⊤ ∈ RTp·m.

C	Y

Let  and  denote constraint sets on the input and output uf and yf respectively. Given a set of stage cost functions ci and past trajectory of the system, we are interested in resolving the following problem in a receding horizon fashion.

min

uf,yf

Tf−1

Σ

ci(uf,i, yf,i)	(3a)

i=0

yp

s.t.  yf = Fp up + Ffuf,	(3b)

(uf, yf) ∈ C × Y.	(3c)

∈

i=0

{	}

Without access to the model (2), we will need to reconstruct it from offline data. Let vT = v(i) T −1 denote measurements of a signal v with length T Z≥1. We define the Hankel matrix of depth L of vT as

—

 v(0)	v(1)	. . .  v(T — L — 1)

H (v

) = 

v(1)	v(2)	. . .	v(T	L)



(4)



L  T	.	.	. . .	.	

v(L — 1)  v(L)  . . .	v(T — 1)

Note that each column of the Hankel matrix contains a trajectory of length L.

Definition A.1 (Persistency of Excitation). For a given sequence vT , we call the sequence persistently exciting of order L if HL(vT ) has full row rank.

Using persistently exciting inputs, Willems’ Fundamental Lemma allows us to parametrize all finite-length trajectories of a controllable linear system.

·

Lemma A.2 (The Fundamental Lemma Willems et al. (2005)). Consider a controllable linear time-invariant system of order n. Given an input sequence uT which is persistently exciting of order L m + n and the corresponding output sequence yT , then there exists g such that any admissible trajectory (u, y) of length L can be expressed as a linear combination

HL(yT )

y

HL(uT ) g =  u .	(5)

DeePC is based on the use of this implicit predictor (5) as a proxy for the explicit model representation in (2). In an online manner, DeePC matches the “past” data with the most recently seen inputs and outputs, and uses the “future” data to match a prediction over which we optimize. This results in a

receding-horizon predictive controller based purely on data:

Σ

Tf−1

min

uf,yf,g

ci(uf,i, yf,i) + r(g)	(6a)

i=0

u





HTp+Tf (yT )

H	(u )	up

yp

yf

s.t.

Tp+Tf	T	g =

f  ,	(6b)

(uf, yf) ∈ C × Y.	(6c)

Here, r(g) denotes suitable regularizers on g Dörfler et al. (2023) which improve robustness of the controller in the presence of noise. The implicit predictor in (5) can readily be extended to affine systems as follows Martinelli et al. (2022); Berberich et al. (2022)

	

Ht(uT )

	

Ht(yT )  g =

1⊤

u

" #

y  .	(7)

1

### From Implicit to Explicit Predictor

Partitioning the predictor (7) into past and future states results in

U

u

 Up 	up

f

f

 Yp  g = yp ,	(8)

1⊤

1

 Yf 	 yf 

which is an implicit predictor for the future measurement trajectory yf given past measurements yp, the corresponding past input sequence up and a future input sequence uf. Using the partitions

 Up 	up

U

u



1

z



p

1⊤

yp 

H :=  Y f  , z :=  f  ,	(9)

we can eliminate g from the implicit predictor in Equation (8) and write the equivalent explicit predictor

yf = YfH†z + YfNH g0,	(10)

z	z

z

where A† denotes the pseudo-inverse of A, NH is the null-space projection matrix of Hz and g0 is

z

an arbitrary perturbance to the least squares solution g = H†z. Assuming Hz has full row rank, then

the least squares solution (i.e. NHz g0 = 0) is

z

z

z

yf = YfH⊤ H H⊤ −1 z.	(11)

# SQP-MPC

Nonlinear MPC tries to optimize a cost function subject to the state evolution of a nonlinear system of the form

yf(k) = f (up(k — 1), uf(k), yp(k — 1)) .	(12)

If we had access to f , then we could use the receding-horizon optimal control problem

Σ

Tf−1

min

uf,yf

ci(uf,i, yf,i)	(13a)

i=0

s.t.  yf = f (up, uf, yp) ,	(13b)

(uf, yf) ∈ C × Y	(13c)

f

to obtain an optimal control input sequence u⋆. One method of solving a nonlinear optimal control problem of this form is using SQP, which repeatedly solves a linearization of the nonlinear problem (13a), resulting in subproblem (14). The solution of said subproblem is then used to update the estimate of the optimal open-loop solution (u˜f, y˜f).

min

∆uf,∆yf

Tf −1

Σ

i=0

⊤

∆y

f,i

⊤

∆u

f,i

1 H˜i

∆yf,i

"	#

∆uf,i 1

(14a)

∆yp

∆zp =	 ,

s.t.  ∆yf = Fp ∆up + Ff∆uf,	(14b)

up(k) — u˜p	(14c)

yp(k) — y˜p

(∆uf,i, ∆yf,i) ∈ C × Y	6i ∈ {0, . . . , Tf — 1} ,	(14d)

where the symbols Fp :=

p

∂

∂hu⊤

⊤ f (up, yp, u˜f) and Ff := ∂

y

∂u˜f

⊤

i

p

f (up, yp, u˜f) indicate the respec-

tive Jacobians of system (12) evaluated at the current solution estimate (up, yp, u˜f, y˜f). The full SQP algorithm can be summarized as follows Diehl et al. (2002)

Algorithm 4 SQP-MPC (IO Representation)

1: function SQP-MPC(zp)

2:	while not converged do

3:	linearize dynamics, constraints and cost at (zp, u˜f, y˜f)

4:	Solve Equation (14)

→

5:	(u˜f, y˜f)	(u˜f, y˜f) + (∆uf, ∆yf)

6:	end while

7:	Return u˜f,0

8: end function

### Equivalence between Select-DPC and SQP-MPC

Ð

Select-DPC has a very natural interpretation as solving a nonlinear MPC problem using SQP, where the analytical Jacobian computation is replaced by a data-driven approach of estimating the Jacobian of the input-output behavior from data. Specifically, one can select data points “close” to the current linearitation point (given by the open loop solution), resulting in an equivalent, data-driven approximation of Fp and Ff in Equation (14) that approaches the analytical Jacobians assuming sufficient amounts of data in the neighbourhood around the linearization point (i.e. card( ) tending to infinity).

# Isomap

Isomap, short for Isometric Mapping, is a nonlinear dimensionality reduction technique introduced by Tenenbaum Tenenbaum et al. (2000). It can be seen as an extension of classical Multidimensional Scaling (MDS) where, instead of using Euclidean interpoint distances, geodesic distances between the data points are preserved. These geodesic distances are estimated using shortest distances on a connected graph which connects neighboring data points. These shortest paths are then arranged in a distance matrix D. Isomap then finds an embedding by applying classical MDS. Computing the distance of a new query data point to the data set in the embedding dimension is done by first linking the new data point into the neighborhood graph and then projecting the data point into the embedding space.

Isomap is able to capture the overall nonlinear nature of the data manifold and is guaranteed to produce a global optimizer of its reconstruction loss. Furthermore, due to preserving geodesic distances, points close in the input space are also close in the embedding space, resulting in interpretable results. This is especially relevant for Select-DPC which is build upon the idea of selecting data points according to their associated distance to a query point.

×103

The  relative  contrast  of  a  data  set

Relative Contrast ∆

Emb. Dim. (dashed)

∆ = (dmax — dmin)/dmin,  with  dmax/min  in-

dicating the maximum and minimum norm values of the data set respectively, is a helpful surrogate to determine how distinct each data point appears under a given distance metric. Figure 7 shows the relative contrast of a data set collected from the rocket simulator in Section 4 as a function of trajectory dimensionality when embedded using Isomap. It compares relative contrast given a fixed embedding dimensionality as well as a varying one, which increases

40

20

0	200	400

Isomap (nd = 75) Isomap (nd varying)

L1

Trajectory Dimensionality

1.0

0.5

0.0

according to n + Tf · m, to the relative contrast  Figure 7: Relative contrast with Isomap embedding

of the L1 norm. Both Isomap methods have better relative contrast compared to L1 selection.

compared to L1 selection.

While a varying embedding dimensionality offers greater relative contrast for small trajectory lengths, we encounter diminishing returns for large trajectory lengths.

### Isomap Parameter Selection

·

Isomap comes with a set of hyper parameters that need to be selected for the data set at hand. Specifically, the number of closest data points that should be considered as neighbors during the neighborhood graph construction and the dimensionality of the embedding space. Figure 8 shows the reconstruction error of the data sets as a function of number of graph neighbors used during adjacency graph construction and number of dimensions of the embedding space. The graphs show a minimum around 10 graph neighbors while we can observe diminishing returns from increasing the embedding dimensionality from 64 to 128 which is consistent with the formula Tf m + n which suggests that the locally linear system has a dimensionality of 96.

Furthermore, note the sharp increase in reconstruction error as the number of graph neighbors increases after the local minimum. This demonstrates the effect of bad hyperparameter selection which can lead to short-circuiting or over-connectedness in the graph, leading to worse embedding performance as the local manifold structure is lost Tenenbaum et al. (2000).

Dim 16

Dim 32

Dim 64

Dim 128

2 × 105

## Isomap Embedding Error Random Walk

## IID

Reconstruction Error

105

6 × 104

4 × 104

101	102

# Graph Neighbors

101	102

# Graph Neighbors

Figure 8: Isomap reconstruction error as a function a embedding dimensionality and number of neighbors used in the adjacency graph.

# Experiment Hyperparameters

All experiments were performed using a quadratic tracking cost of the form

ci(uf,i, yf,i) =  yf,i — yr  Q +  uf,i  R,	(15)

where Q and R are positive (semi-)definite cost matrices and yr is a reference setpoint. Furthermore, for the regularization term r(g) we use 1-norm and projection regularization Markovsky et al. (2023) with weights λ1 and λΠ respectively. The following tables shows the hyperparameters for the three sets of simulations.

Parameter

Q R

λ1 λπ

Value

diag ([40  20  20  1  3000  30])

diag ([10  10  10])

0

5000

Table 1: Hyperparameters for the rocket simulation environment.

| Parameter |  | Value |
| --- | --- | --- |
| Q | diag ([0 | 0 0 0 40 000 40 000 10 10]) |
| R |  | diag ([10 10]) |
| λ1 |  | 10 |
| λπ |  | 10 000 |

Table 2: Hyperparameters for the reacher simulation environment.

| Parameter |  |  | Value |
| --- | --- | --- | --- |
| Q | diag ([5 | 0 | 10 000 0.1 0.1]) |
| R |  |  | 1 |
| λ1 |  |  | 50 000 |
| λπ |  |  | 0 |

Table 3: Hyperparameters for the cart-pole simulation environment.